Table of Contents

# 1	Road Transport Model Overview

Purpose note This is a plain-English overview of the road transport model in leap_road_model. For implementation details, module sequencing, and output tables, see road_transport_model_detailed.md. For Module 1 data sourcing and the interface contract, see the road-model guide in the sibling road_model_inputs_interface repo.

End-to-end road model workflow

Primary reference for the full end-to-end workflow. Some implementation detail is not shown.

Road transport model - researcher detail

Simplified illustration of the stock-flow modelling logic.

The road transport model is part of the APERC LEAP modelling system. It estimates future road transport energy demand for passenger and freight vehicles, including cars, motorcycles, buses, light commercial vehicles, and trucks. Its purpose is to translate assumptions about vehicle stock, sales, technology uptake, mileage, and fuel economy into energy demand by fuel.

The model is built around a simple idea:

Energy use = vehicle stock x mileage x fuel economy

In practice, the model contains many vehicle types, drive types, and fuels. Users do not need to understand every internal calculation to follow the logic. At a high level, the model asks:

- how many vehicles are on the road;

- what kinds of vehicles they are;

- how quickly older vehicles are replaced; and

- what fuels and technologies new vehicles use.

The workflow begins outside LEAP. This is not just a data-preparation step. A significant part of the road model is prepared before LEAP because stock, sales, retirements, and energy need to stay internally consistent.

Inputs are first prepared in a browser-based interface. The interface gives users one place to review, update, test, and document the main assumptions that describe the road transport system. These inputs fall into three broad groups.

First, the interface is used to update base-year evidence. This includes estimates of vehicle stock, mileage, and fuel economy. It also includes reconciliation settings such as weights, bounds, and adjustment rules, which help align the detailed vehicle data with the ESTO energy balances in the base year.

Second, the interface is used to prepare structural projection assumptions that are not intended to be edited directly in LEAP. These include the overall road stock and sales trajectory, projected vehicle-type shares such as cars, motorcycles, buses, and trucks, survival and vintage profile assumptions, freight stock-growth assumptions, and PHEV utilisation shares. These assumptions are important because they affect the stock-flow pathway itself. Changing them can affect sales, retirements, stock, and energy at the same time, so they are better prepared through the pre-LEAP workflow.

Third, the interface can be used to pre-set and test variables that may later be adjusted in LEAP. These include technology sales shares, mileage adjustment factors, fuel economy adjustment factors, fuel or device shares, and some scrappage settings. This means users can work in one place if they prefer: they can update base-year evidence, test projection assumptions, inspect the implied stock and energy outcomes, and then export a LEAP-ready package.

The Python workflow then converts these inputs into the structure that LEAP expects, creates import workbooks, and can run optional checks and simplified pre-LEAP simulations. These checks are not the official projection result, but they help users see whether their assumptions produce sensible stock, sales, retirement, and energy outcomes before the full LEAP model is run.

LEAP remains the official projection platform. Once the road transport inputs have been prepared and imported, LEAP calculates the official road transport energy projections as part of the wider APERC energy system model. This matters because road transport does not sit by itself: its fuel demand affects electricity generation, refining, biofuels, hydrogen, oil product demand, imports, exports, and the overall energy balance.

The Python workflow and dashboards should therefore be understood as support tools around LEAP. They help users prepare inputs, identify problems, compare assumptions, and understand model behaviour before and after running LEAP. They are especially useful because the road model is detailed and data-heavy, and because some stock-flow checks are easier to manage outside LEAP than inside it.

## 1.1	What is prepared before LEAP?

Some assumptions are prepared through the browser interface and Python workflow before they are imported into LEAP. These are the assumptions that define the overall stock-flow pathway or the base-year calibration. They include:

- the overall road stock and sales trajectory;

- projected vehicle-type shares, such as cars, motorcycles, buses, and trucks;

- survival and vintage profile assumptions;

- freight stock-growth and elasticity settings;

- PHEV utilisation shares;

- base-year stock, mileage, and fuel economy estimates;

- reconciliation weights, bounds, and adjustment settings.

These assumptions are structural. For example, changing the share of cars, buses, or motorcycles is not just a simple split in LEAP: it can change total stock, total sales, retirements, and energy use. Similarly, survival and vintage profiles affect how quickly vehicles retire and how many new vehicles are needed to maintain the target stock pathway.

PHEV utilisation is also better prepared before LEAP because the calculation needed to convert a utilisation assumption into a fuel split can be complicated. The final fuel or device shares may appear in LEAP, but the underlying utilisation assumption is easier to manage in the pre-LEAP workflow.

These inputs are pre-set using the best available evidence and expert judgment, but users can review and adjust them if needed. The model is designed so that users can start with the most important variables and then explore more detailed assumptions if they want to go further. The goal is to keep the model flexible and transparent without requiring users to adjust every assumption manually.

## 1.2	What can be adjusted in LEAP?

Some road transport assumptions can be adjusted directly in LEAP because they affect energy use or technology choice without necessarily changing the overall stock-flow structure. These include:

- mileage adjustment factors;

- fuel economy adjustment factors;

- technology sales shares, such as ICE, BEV, PHEV, and FCEV shares;

- fuel or device shares where the LEAP scenario requires them;

- scrappage or accelerated retirement settings, although these still need testing because they may interact with stock targets.

These variables are useful for scenario testing because they let users explore technology uptake, efficiency improvement, fuel switching, and changes in vehicle use within LEAP.

In practice, some of these same variables can also be pre-set and tested in the browser interface and Python workflow before they are imported into LEAP. This is useful when users want to check the effect of their assumptions in one place before running LEAP.

The interface also supports saving progress and moving data into Excel when that is faster for manual review or editing.

For road transport, the main principle is simple: LEAP is the official projection platform, but the road model is partly prepared before LEAP. Base-year evidence, reconciliation settings, and some projection assumptions are managed through the interface and Python workflow so that the stock-flow logic remains consistent. LEAP then uses those prepared inputs to calculate the official projection within the wider APERC model.

The model also needs to remain consistent with historical energy data. For the base year, road transport energy use should align with the ESTO energy balances. That means that even when detailed vehicle data are available, some calibration or reconciliation may still be needed. If the estimated number of vehicles, their mileage, and their fuel economy imply too much or too little fuel use, the model adjusts within reasonable bounds so that the base year remains consistent with observed energy totals.

In short, the road model acts as a bridge between transport assumptions and LEAP energy projections. The browser interface helps users update base-year data, set reconciliation rules, prepare structural projection assumptions, and test LEAP-adjustable variables. The Python workflow turns those assumptions into LEAP-ready inputs and quality checks. LEAP then produces the official energy projection within the wider APERC model. The goal is to keep a detailed road transport model while making it easier for users to understand, test, and explain.

# 2	Road transport model methodology guide

Purpose note
This document explains the methodology of the road transport model in leap_road_model. It is intended for researchers, reviewers, and external readers who need to understand how the model works without reading the detail required for using or managing the code. It is more detailed than the front-page overview, but it does not replace road_transport_model_modeller_guide.md, which remains the modeller guide.

## 2.1	Contents

- What the road model is for

- Road branch structure in LEAP

- Main variables and how they interact

- Why some road preparation happens before LEAP

- Overall model workflow

- Base-year setup

- Passenger road stock projection

- Freight road stock projection

- Sales, survival, vintage profiles, and turnover

- Technology sales shares and LEAP scenario assumptions

- Base-year fuel allocation and reconciliation

- What researchers can adjust before LEAP

- What researchers can adjust in LEAP

- Python simulated outputs, LEAP, and official results

- Open methodology items for Codex / developer follow-up

End-to-end road model workflow

Primary reference for the full end-to-end workflow. Some implementation detail is not shown.

Road transport model â€” researcher detail

Simplified illustration of the stock-flow modelling logic.

## 2.2	1. What the road model is for

The road transport model estimates road transport energy demand by fuel. It covers passenger and freight road transport, including LPVs, motorcycles, buses, trucks, and light commercial vehicles.

The model translates transport assumptions into energy demand. The main questions it answers are:

- how many vehicles are on the road;

- what types of vehicles they are;

- what technologies they use;

- how far they travel;

- how efficient they are;

- what fuels they consume; and

- how quickly old vehicles are replaced by new vehicles.

The model is detailed because road transport is one of the demand sectors where bottom-up stock-flow data can explain energy use relatively well. If vehicle stock, mileage, fuel economy, fuel shares, and technology shares are reasonable, the implied road fuel demand can often be close to observed energy totals.

However, the data are not always complete or consistent. Some economies have strong vehicle registration data, but weaker mileage or fuel economy data. Others have reasonable aggregate road fuel demand, but limited vehicle-level detail. For this reason, the model combines bottom-up transport assumptions with reconciliation to the official ESTO energy balances.

The workflow combines three parts:

- a browser-based interface for reviewing and updating assumptions;

- a Python workflow for preparing, checking, and reconciling inputs; and

- LEAP as the official projection platform.

The Python workflow is not intended to replace LEAP. It prepares the road model so that the stock-flow inputs are coherent and reviewable before the official projection is run in LEAP.

## 2.3	2. Road branch structure in LEAP

The road branch hierarchy runs:

transport type â†’ vehicle type â†’ drive type + size where relevant â†’ fuel type

Each leaf in the tree below is a LEAP branch.

Demand
  Passenger road
    LPVs                                 [sizes: small, medium, large]
      ICE <size>
        Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG
        Biogasoline, Biodiesel, Biogas, Efuel
      HEV <size>
        Motor gasoline, Gas and diesel oil, Biogasoline, Biodiesel, Efuel
      EREV <size>
        Electricity, Motor gasoline, Biogasoline, Efuel
      PHEV <size>
        Electricity, Motor gasoline, Biogasoline, Efuel
      BEV <size>
        Electricity
      FCEV <size>
        Hydrogen
    Motorcycles
      ICE
        Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG
        Biogasoline, Biodiesel, Biogas, Efuel
      BEV
        Electricity
      FCEV
        Hydrogen
    Buses
      ICE
        Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG
        Biogasoline, Biodiesel, Biogas, Efuel
      BEV
        Electricity
      FCEV
        Hydrogen
  Freight road
    Trucks                               [sizes: medium, heavy]
      ICE <size>
        Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG
        Biogasoline, Biodiesel, Biogas, Efuel
      BEV <size>
        Electricity
      FCEV <size>
        Hydrogen
    LCVs
      ICE
        Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG
        Biogasoline, Biodiesel, Biogas, Efuel
      PHEV
        Electricity, Motor gasoline, Biogasoline, Efuel
      BEV
        Electricity
      FCEV
        Hydrogen

The intention is to provide detail where it is useful, not to make every branch equally detailed. LPVs receive the most detail because they are usually the largest part of the fleet, have better data, and are where many technology-transition assumptions are most policy-relevant. Smaller or less data-rich branches are kept simpler where extra detail would add work without improving the result. At the same time, the model still needs enough detail to represent the different needs of APEC economies. Reducing detail is useful for keeping the model understandable and manageable, but it has to be weighed against the need to represent economies with different fuels, technologies, and vehicle structures.

This hierarchy matters because variables are defined at different levels, which is a result of model design decisions, which was sometimes influenced by the way LEAP works.

This means a change at one level can affect many branches below it. For example, changing BEV sales share under LPVs affects future BEV stock, electricity demand, ICE stock share over time, and liquid fuel demand. Changing mileage at a fuel branch affects energy use for that branch, but does not directly change stock or sales.

Current scope rules (important when comparing to the 9th edition and earlier 10th edition LEAP models):

- HEV and EREV are LPV-only.

- Truck PHEV is out of scope.

- LPVs use small, medium, and large size labels.

- Trucks use medium and heavy size labels where truck-size splits are needed.

- Fuel Economy is the canonical Module 1 efficiency variable. Final On-Road Fuel Economy can be accepted only as a legacy input alias.

The changes were generally intended to The same vehicle/drive/size matrix should be used by Module 1 validation and Module 2 branch generation. Module 1 rows outside this matrix should be rejected or explicitly recategorized before export; Module 2 should not create branches outside this matrix during skeleton generation.

## 2.4	3. Main variables and how they interact

This section introduces the main variables used in the road model. Later sections explain them in the order that the model uses them.

### 2.4.1	Stock

Stock is the number of vehicles on the road. Stock can be described at different levels: total passenger road stock, LPV stock, BEV stock, or fuel-specific stock implied by Device Shares.

Stock is central because more vehicles usually means more energy use, unless offset by lower mileage, better efficiency, or a shift to more efficient technologies.

### 2.4.2	Sales

Sales are new vehicles entering the fleet in a given year. Sales determine how quickly the fleet changes. A future BEV sales share of 80% does not mean that 80% of the fleet is immediately BEV. It means that 80% of new sales are BEV, and the stock share changes gradually as older vehicles retire.

### 2.4.3	Retirements

Retirements are vehicles leaving the fleet. Retirements are mostly determined by survival curves and vintage profiles. Additional scrappage can also be used where a policy causes vehicles to retire earlier than normal.

Retirements matter because they create the need for replacement sales. Faster retirement can accelerate technology turnover, but it can also increase total sales and change the timing of stock changes.

### 2.4.4	Survival and vintage profiles

A survival profile describes how likely a vehicle is to remain on the road at each age. A vintage profile describes the age distribution of the fleet in the base year.

These profiles connect the stock target to sales. If the fleet is young and vehicles survive for a long time, fewer replacement sales are needed. If the fleet is old or vehicles retire quickly, more sales are needed to maintain the same stock.

### 2.4.5	Mileage

Mileage is the average distance travelled per vehicle per year. Higher mileage increases energy use without changing the number of vehicles. Mileage can represent travel demand, vehicle utilisation, modal shift effects, or other behavioural and operational changes.

### 2.4.6	Fuel economy / efficiency

Fuel economy describes how much energy is needed to travel a given distance. In the current workflow, fuel economy may be represented as distance per unit of energy, such as km/GJ. Under that convention, higher fuel economy means lower energy use.

This is important for interpreting scalars and correction factors: increasing a km/GJ efficiency value reduces energy demand, while increasing an MJ/km energy-intensity value would increase energy demand.

### 2.4.7	Sales shares

Sales shares split new sales across vehicle types, technologies, and sometimes sizes. They are the main way that technology uptake enters the stock-flow model.

Sales shares affect future energy demand indirectly. They first change new sales, then stock, then activity by technology, and finally energy use by fuel.

### 2.4.8	Stock shares

Stock shares describe how total stock is split across vehicle types or technologies. In the base year, they help define the starting fleet structure. In projection assumptions, some stock shares can define structural pathways, such as the split between LPVs, motorcycles, and buses.

Stock shares are not always safe to edit directly in LEAP because they can change the stock-flow pathway that sales and turnover need to satisfy.

### 2.4.9	Device Shares

Device Shares split a technology branch across fuels. For example, a PHEV branch may consume both electricity and liquid fuel. An ICE branch may have gasoline, diesel, LPG, natural gas, or biofuel-related fuel branches depending on the branch structure.

Device Shares are closely linked to fuel allocation and reconciliation. They should generally be interpreted as the modelâ€™s fuel split within a branch, not always as a literal count of separate vehicle technologies.

### 2.4.10	Energy

At the simplest level:

energy = stock Ã— mileage Ã— energy use per kilometre

Where fuel economy is represented as km/GJ, the same relationship is written as:

energy = stock Ã— mileage / fuel economy

The model uses this relationship at the branch level, then aggregates across branches to produce road energy demand by fuel.

## 2.5	4. Why some road preparation happens before LEAP

LEAP remains the official projection platform, and many important interactions still happen inside LEAP. The reason part of the road model is prepared before LEAP is not that LEAP cannot represent stock-flow behaviour. The reason is that several of the road model inputs are difficult to prepare, calibrate, and review directly inside LEAP.

The main pre-LEAP preparation tasks are:

- the overall road stock and sales trajectory;

- survival and vintage profile assumptions;

- PHEV utilisation shares;

- base-year stock, mileage, and fuel economy estimates; and

- base-year reconciliation to ESTO road fuel totals.

These tasks involve linked calculations, source-data cleaning, judgement about uncertain inputs, and checks against historical energy balances. Preparing them outside LEAP simplifies the modelling process because researchers can review the main assumptions in a focused interface while Python prepares the dependent LEAP inputs consistently.

This should not be understood as a separation where Python does all interactions and LEAP simply stores the result. Some interactions occur before LEAP because they are easier to prepare there, while others occur in LEAP during the official stock-turnover projection. The pre-LEAP workflow is therefore a preparation and QA layer around LEAP, not a replacement for LEAP.

## 2.6	5. Overall model workflow

The road model workflow follows the same broad order as the modelling logic.

- Prepare base-year evidence.
The workflow starts with stock, mileage, fuel economy, fuel eligibility, observed road fuel demand, sales shares where available, and supporting macroeconomic data.

- Project road stock targets.
Passenger stock is projected through a vehicle-equivalent ownership and saturation method. Freight stock is projected through a GDP-elasticity method.

- Convert stock targets into sales and retirements.
Survival curves and vintage profiles are used to determine how many vehicles remain each year and how many new sales are needed to reach the stock target.

- Prepare or seed sales shares.
Base-year sales shares initialise the model, while future sales-share assumptions are mainly scenario levers. Some may be seeded before LEAP, but a large part of technology uptake is expected to be reviewed and adjusted in LEAP.

- Allocate fuels and reconcile the base year.
The model calculates initial branch energy from stock, mileage, and fuel economy, allocates ESTO fuel totals to eligible branches, adjusts uncertain inputs within bounds, and calculates Device Shares.

- Export LEAP-ready inputs.
The prepared inputs are imported into LEAP, where the official projection is run as part of the wider APERC energy system.

- Use dashboards and simulated outputs for QA.
Python can produce simulated outputs and diagnostics to help users understand likely effects, but LEAP remains the official projection.

## 2.7	6. Base-year setup

The base year is the starting point for the road model. It needs to be consistent in two ways:

- it should reflect the best available transport evidence; and

- it should align with the official ESTO road energy balances.

The starting data usually include vehicle stock, mileage, fuel economy, fuel eligibility, base-year sales shares, survival curves, vintage profiles, population, GDP, and ESTO road fuel totals.

The model first uses the transport data to build a bottom-up representation of the base-year fleet. This means it estimates energy use from vehicles, distance travelled, and efficiency. It then compares the implied energy demand with ESTO fuel totals during the reconciliation step.

The base-year setup is not just a data-loading step. It defines the starting fleet structure for the projection. If the base-year stock mix, mileage, efficiency, or fuel allocation is wrong, the projection can still run, but the future results may be difficult to interpret.

The base-year setup also creates the link between the detailed road model and the wider LEAP energy system. Since the wider APERC model is built around energy balances, the road model must begin from a base year that is consistent with those balances.

## 2.8	7. Passenger road stock projection

Passenger road stock is projected using a motorisation approach. The model estimates how passenger vehicle-equivalent ownership changes with population, GDP per capita, recent passenger road energy growth, and saturation conditions.

The key issue is that not all passenger vehicles represent the same transport capacity. One LPV, one motorcycle, and one bus should not be treated as equivalent physical units. A bus can represent many LPV-equivalent units of passenger mobility or ownership demand, while a motorcycle represents a smaller unit.

The model therefore uses vehicle-equivalent ownership. Physical passenger vehicle stocks are converted into a weighted stock measure before the overall passenger motorisation pathway is projected.

At a high level, the passenger method is:

- convert base-year LPVs, motorcycles, and buses into vehicle-equivalent stock;

- divide by population to estimate base-year vehicle-equivalent ownership per person;

- compare this with a long-run saturation level;

- estimate a GDP-per-capita income elasticity from historical passenger road energy growth and historical GDP-per-capita growth;

- project ownership with future GDP-per-capita growth, damping that growth as ownership approaches saturation;

- convert the projected vehicle-equivalent stock back into physical vehicle-type stock targets; and

- pass those stock targets to the sales and turnover calculation.

The saturation level represents the long-run vehicle-equivalent ownership level that the economy is expected to approach. Economies with low current ownership may continue to grow toward saturation as GDP per capita rises. Economies that are already close to saturation should not be forced to keep increasing ownership simply because historical energy grew.

For economies that are not saturated, the model uses passenger road energy as the activity proxy and compares its historical growth with historical GDP-per-capita growth. The resulting income elasticity is bounded, multiplied by the reviewed passenger stock growth-rate adjustment, and then checked against the same bounds again. If the historical data are insufficient or GDP-per-capita growth is near zero, the model uses the default passenger income elasticity.

For each projection year, GDP-per-capita growth is applied to the previous year’s vehicle-equivalent ownership level. The growth effect is multiplied by the remaining distance to saturation, expressed as 1 - current motorisation / saturation motorisation. This keeps the projection responsive to future income growth while making ownership growth fade as saturation is approached.

For economies that are already saturated, the model holds vehicle-equivalent ownership broadly flat. Physical stock can still change if population changes or if the vehicle-type split changes, but the model is no longer assuming structural growth in ownership per person.

The passenger vehicle-type split matters because it changes how the aggregate vehicle-equivalent stock is converted back into physical vehicles. A shift toward buses can reduce the number of physical vehicles needed for a given passenger vehicle-equivalent pathway. A shift toward motorcycles can increase or decrease physical counts depending on the relative weights used and the starting fleet composition.

This is why passenger vehicle-type assumptions are better prepared before LEAP. They are not just cosmetic branch shares. They affect the stock target, sales requirements, and eventual energy use.

### 2.8.1	Passenger stock interaction example

Suppose an economy has rising passenger vehicle-equivalent ownership. If most of the increase is allocated to LPVs, physical vehicle stock may rise strongly. If some of the increase is allocated to buses, the physical number of vehicles may rise less, because each bus represents more vehicle-equivalent capacity. That then changes the number of sales required, the future technology mix, and total energy demand.

This means vehicle-type shares, vehicle-equivalent weights, saturation assumptions, and passenger growth adjustments should be reviewed together rather than treated as separate inputs.

## 2.9	8. Freight road stock projection

Freight road stock is projected differently from passenger road stock. Freight does not use a passenger-style ownership saturation curve. Instead, the current method links freight stock growth to GDP growth through a GDP elasticity.

The elasticity answers a simple question:

If GDP grows by 1%, how much should freight vehicle stock grow?

At a high level, the freight method is:

- estimate a freight GDP elasticity from historical freight road energy and GDP trends where possible;

- clamp the estimated elasticity within reasonable bounds;

- apply any reviewed adjustment to that bounded elasticity and check it against the same bounds again;

- project total freight stock using GDP growth; and

- split the projected stock between trucks and LCVs using base-year proportions.

The freight activity proxy is historical freight road energy growth. Unlike the passenger method, the freight method estimates compound endpoint growth rates over the filtered lookback window: one for freight road energy and one for GDP. It then divides freight energy growth by GDP growth to estimate a GDP elasticity. If either series has fewer than two usable observations, if the first usable value is not positive, or if GDP growth is effectively zero, the default elasticity is used instead.

This makes freight stock growth responsive to economic activity while avoiding a more complex freight tonne-kilometre model. The method is intentionally simpler than the passenger method because freight data are often weaker, and because the current model does not yet include a detailed mode-choice or logistics-demand structure.

The truck/LCV split is currently treated as a fixed structural split based on the base year. This means the model focuses on the overall growth of freight road stock rather than projecting a detailed shift between truck and LCV activity. If stronger evidence becomes available in the future, this could be expanded, but for now the simpler method keeps the freight model transparent and easier to review.

### 2.9.1	Freight stock interaction example

If GDP grows quickly and the freight elasticity is high, freight stock grows quickly. That increases future sales requirements and energy demand, unless offset by efficiency improvement, lower mileage, or a shift to more efficient technologies. If the elasticity is low, the freight fleet grows more slowly even if GDP rises.

This makes the freight elasticity one of the most important freight road assumptions. It should be reviewed against historical energy trends, economy structure, and whether freight demand is expected to decouple from GDP growth.

## 2.10	9. Sales, survival, vintage profiles, and turnover

After passenger and freight stock targets are prepared, the model needs to determine sales and retirements. This is the stock-flow part of the model.

The basic accounting identity is:

ending stock = surviving prior stock + new sales - additional retirements

A stock target says how many vehicles should be on the road in a future year. The model then uses survival and vintage assumptions to estimate how many vehicles from previous years remain. The difference between the target stock and surviving stock determines how many new sales are needed.

If surviving stock is below the target, new sales fill the gap. If surviving stock is already above the target, the model needs to handle this carefully. This can happen when the target stock is flat or declining but the existing fleet survives for a long time.

Turnover is important because it controls the speed of technology change. Even if new sales become mostly BEV, the existing ICE fleet remains until it retires. A younger fleet with long lifetimes changes slowly. An older fleet or a policy with additional scrappage changes more quickly.

### 2.10.1	Survival curves

A survival curve describes the probability that a vehicle is still operating at each age. A curve with long survival keeps more older vehicles in the fleet. A curve with shorter survival causes faster retirements and higher replacement sales.

### 2.10.2	Vintage profiles

A vintage profile describes the base-year age distribution of the fleet. Two economies can have the same total stock but very different age structures. If one fleet is much older, it may have higher near-term retirements and therefore faster stock turnover.

### 2.10.3	How profiles are checked and adjusted

Survival and vintage profiles are not just imported as decorative assumptions. Module 4 normalises survival curves and vintage profiles, aligns them to the same age range, and uses them in cohort stock-flow accounting.

If lifecycle calibration factors are available, the workflow can scale survival curves to meet turnover-rate bounds and then re-derive vintage profiles from the calibrated survival curves. A base-year fleet age shift can also be applied before the stock-flow calculation. These adjustments are recorded in the Module 4 lifecycle outputs and exported through the lifecycle profile workbooks.

Where surviving vehicles exceed the target stock, the model does not produce negative sales. It scales the surviving cohorts down to the target, records stock_above_target = True, and stores the applied scale factor. This is important for economies or vehicle types where the stock path is flat or declining while the existing fleet remains on the road for a long time.

### 2.10.4	Additional scrappage

Additional scrappage represents early retirement beyond normal survival. It can be used to test policies that retire older or inefficient vehicles faster. However, scrappage affects more than energy demand. It also affects sales, stock accounting, technology uptake, and possibly the timing of fleet replacement. It should therefore be tested carefully before being treated as a standard scenario lever.

### 2.10.5	Turnover interaction example

Consider two economies with the same BEV sales share pathway. In the economy with faster retirements, BEVs enter the total stock more quickly because more old vehicles are replaced each year. In the economy with slower retirements, the same BEV sales share has a slower effect on total stock and fuel demand. This is why survival and vintage assumptions are not background details; they shape how quickly sales-share assumptions affect energy outcomes.

## 2.11	10. Technology sales shares and LEAP scenario assumptions

Sales shares allocate new sales across vehicle types, drive technologies, sizes, and sometimes fuels. They are one of the main ways that technology uptake enters the road model.

For example, in the LPV branch, sales shares can determine how new sales are divided between ICE, HEV, PHEV, BEV, and FCEV. In freight, they can determine how new truck or LCV sales are divided between ICE, BEV, FCEV, and any other included technologies.

Sales shares affect the model through time. They do not immediately change the whole fleet. Instead:

sales shares â†’ new sales by technology â†’ stock by technology â†’ energy by fuel

This delay is central to stock-flow modelling. A rapid increase in BEV sales may have a modest effect on total electricity demand at first if most of the existing fleet is still ICE. Over time, as old vehicles retire and BEVs accumulate in the stock, the effect becomes larger.

A large part of technology uptake is intended to be handled in LEAP. The pre-LEAP interface can provide preliminary sales-share assumptions and allow users to observe simulated outputs, but the official projection depends on how LEAP applies its own stock-turnover calculations, scenario expressions, and calculation order. For that reason, pre-LEAP technology results should be described as simulated outputs or QA outputs, not official projections.

Base-year sales shares are used to initialise the stock-flow model. Future sales shares may be seeded from previous Outlook pathways, defaults, or user-edited assumptions, but they should be reviewed in LEAP because they strongly affect the technology pathway.

### 2.11.1	Sales-share interaction example

If BEV sales shares increase while total sales are low, the stock changes slowly because few new vehicles are entering the fleet. If BEV sales shares increase while total sales are high, the stock changes faster. Therefore, technology uptake depends on both sales shares and the total sales pathway.

This is why stock, survival, sales, and sales shares should be reviewed together. Sales shares are important, but they do not determine stock outcomes on their own.

## 2.12	11. Base-year fuel allocation and reconciliation

The base-year reconciliation step is where stock, mileage, fuel economy, fuel allocation, and Device Shares come together. This is why it is better to explain these variables as part of the reconciliation process rather than as completely separate modelling steps.

The purpose of reconciliation is to make the detailed bottom-up road model consistent with ESTO road fuel totals while keeping the underlying transport assumptions plausible. The bottom-up estimate will not always match ESTO because stock, mileage, fuel economy, and fuel split assumptions are all uncertain.

At the simplest level, initial branch energy is calculated as:

vehicle-km = stock Ã— mileage
energy = vehicle-km / fuel economy

where fuel economy is expressed as distance per unit of energy, such as km/GJ. Under this convention, a higher fuel economy value reduces energy use.

The reconciliation process can be understood as the following sequence.

### 2.12.1	Step 1 â€” Calculate initial branch energy

The model first calculates energy for each road branch using the current stock, mileage, and fuel economy assumptions. This gives an initial bottom-up estimate of road energy use by branch and fuel.

This first estimate is useful because it shows what the transport data imply before forcing the result to match the energy balance.

### 2.12.2	Step 2 â€” Handle electricity for BEVs, PHEVs, and EREVs first

Electricity is handled before ordinary liquid fuel allocation because BEVs, PHEVs, and EREVs need to be reconciled against the road electricity total. BEVs are straightforward because they use electricity only. PHEVs and EREVs are more complicated because they split travel between electricity and liquid fuel.

The PHEV utilisation assumption controls the share of PHEV/EREV driving treated as electric. This is difficult to manage directly in LEAP because it affects both electricity demand and liquid fuel demand. The pre-LEAP workflow therefore prepares the implied fuel split so users can review whether the electric/liquid split is plausible.

### 2.12.3	Step 3 â€” Subtract PHEV/EREV liquid fuel from liquid fuel pools

After electricity use is calculated for PHEVs and EREVs, the corresponding liquid fuel use is removed from the relevant ESTO liquid fuel pools before the remaining liquid fuels are allocated to ICE and HEV branches.

This prevents the same liquid fuel being allocated twice. For example, if some gasoline is already assigned to PHEV liquid operation, that gasoline should not also be assigned to ordinary ICE vehicles.

In the current methodology, PHEV and EREV liquid fuel is treated as gasoline-family fuel. Diesel and biodiesel are not assigned to PHEV/EREV branches unless the model is changed deliberately.

### 2.12.4	Step 4 â€” Allocate remaining ESTO fuels to eligible branches

For ordinary fuel allocation, eligibility depends on the vehicle and drive structure. Electricity is handled first for BEV/PHEV/EREV branches, PHEV/EREV liquid fuel is handled separately as gasoline-family fuel, and the remaining liquid or gaseous fuels are allocated to eligible ICE, HEV, and other configured branches.

Fuel allocation is not only a mathematical split. It also embeds transport judgement. The allocation needs to reflect likely fuel use by vehicle type, especially where ESTO reports fuel totals but does not say exactly which road branches consumed that fuel.

The main allocation logic is:

- diesel and biodiesel are allocated to freight ICE vehicles first, with Trucks and LCVs sharing the freight tier in proportion to initial branch energy, before any remaining amount is allowed to spill into passenger vehicles;

- gasoline and biogasoline are allocated to passenger vehicles first, then to LCVs if needed, and are not normally allocated to trucks;

- within a priority group, allocation is proportional to initial branch energy, not raw vehicle stock;

- electricity is handled through the BEV/PHEV/EREV electricity process before ordinary liquid fuel reconciliation;

- hydrogen is allocated only to FCEV branches where relevant; and

- other fuels use their configured eligibility rules and are allocated by energy share across eligible branches unless a reviewed priority rule exists.

The exact fuel allocation order matters because it affects the reconciled stock, mileage, fuel economy, and Device Shares. For example, putting too much diesel into passenger branches may make the base-year model match total diesel demand but produce an implausible vehicle/fuel structure.

### 2.12.5	Step 5 â€” Compare allocated fuel energy with initial branch energy

After fuel has been allocated to eligible branches, the model compares allocated ESTO-consistent fuel energy with the initial bottom-up energy estimate.

This comparison gives the correction required for each branch. If the bottom-up estimate is too low, the model needs to increase implied branch energy through stock, mileage, or fuel economy adjustments. If the estimate is too high, it needs to reduce implied branch energy through those same variables.

### 2.12.6	Step 6 â€” Split the correction across stock, mileage, and fuel economy

The model does not assume that all error comes from one variable. Stock may be uncertain, mileage may be uncertain, and fuel economy may be uncertain. The correction is therefore split across these variables using reconciliation weights.

For example, if the bottom-up estimate is below the allocated fuel energy, the model could:

- increase stock;

- increase mileage;

- reduce km/GJ fuel economy; or

- use a combination of all three.

The reconciliation weights express judgement about which inputs are most uncertain. If stock data are considered strong but mileage data are weak, more adjustment can be directed toward mileage. If mileage and fuel economy are considered reliable but stock data are weak, more adjustment can be directed toward stock.

### 2.12.7	Step 7 â€” Apply bounds to keep adjustments plausible

Reconciliation is not allowed to move values without limit. Bounds are applied so that stock, mileage, and fuel economy do not shift too far from the source evidence.

If the required correction is larger than the bounds allow, a residual gap can remain. That gap should be reported through diagnostics rather than hidden. Large corrections are important review signals because they may indicate weak source data, a wrong fuel allocation rule, missing branches, or a mismatch between transport data and ESTO fuel totals.

If ESTO reports positive energy for a fuel but all eligible branches have zero stock, Module 6 can seed a small positive stock so reconciliation has a branch to adjust. For zero-stock rows that remain in the output, mileage and fuel-economy scalars can be inherited from comparable peer branches so future calculations do not lose adjustment information.

### 2.12.8	Step 8 â€” Recalculate final energy

After stock, mileage, and fuel economy are adjusted, final branch energy is recalculated. The recalculated fuel totals should match ESTO within tolerance unless the bounds prevented full reconciliation.

This recalculated base year becomes the starting point for the projection. If base-year stock changes substantially during reconciliation, the future stock pathway may need to be re-anchored so that the projection starts from the reconciled stock level rather than the original unreconciled estimate.

### 2.12.9	Step 9 â€” Calculate Device Shares

Device Shares are calculated after reconciliation because the fuel split depends on final stock, mileage, fuel economy, and allocated fuel energy.

For single-fuel technologies, Device Share is straightforward. BEV branches consume electricity, and FCEV branches consume hydrogen. For multi-fuel technologies, Device Shares describe the modelled fuel split within the branch.

This is especially important for PHEVs and EREVs, where Device Shares need to reflect the electric/liquid split implied by the utilisation assumption and the reconciliation results.

### 2.12.10	Step 10 â€” Validate and review

The final reconciliation outputs should be checked for:

- whether road fuel totals match ESTO;

- whether stock, mileage, and fuel economy adjustments are within bounds;

- whether Device Shares sum correctly within each branch;

- whether PHEV/EREV utilisation remains plausible;

- whether fuel allocation produced plausible vehicle/fuel combinations; and

- whether any residual gaps or fallback rules need review.

Reconciliation is therefore both a calibration step and a QA step. It makes the model consistent with ESTO, but it also shows where source data or assumptions may need to be revisited.

## 2.13	12. What researchers can adjust before LEAP

The pre-LEAP workflow is used for assumptions that are difficult to prepare directly in LEAP or that need linked calculations before import. These include:

- the overall road stock and sales trajectory;

- base-year vehicle stock;

- base-year mileage;

- base-year fuel economy;

- passenger saturation assumptions;

- vehicle-equivalent weights;

- passenger vehicle-type shares;

- freight GDP-elasticity settings;

- survival and vintage profile assumptions;

- PHEV electric utilisation assumptions;

- reconciliation weights and bounds; and

- notes or metadata explaining overrides.

These variables are prepared before LEAP because they often require several dependent calculations to be updated together. The aim is to simplify the workflow for users while still keeping the assumptions visible and reviewable.

For example, changing survival assumptions affects retirements and sales. Changing passenger vehicle-type shares affects vehicle-equivalent stock allocation. Changing PHEV utilisation affects electricity and liquid fuel demand. Changing reconciliation weights changes how the base-year difference between bottom-up data and ESTO is allocated across stock, mileage, and fuel economy.

## 2.14	13. What researchers can adjust in LEAP

Some variables are suitable for direct LEAP scenario editing because they are useful levers once the stock-flow inputs have been prepared. These include:

- mileage adjustment factors;

- fuel economy or efficiency adjustment factors;

- technology sales shares, such as ICE, BEV, PHEV, and FCEV shares;

- fuel or Device Share settings where relevant.

These variables are useful for scenario design because they let researchers test changes to travel demand, efficiency, technology uptake, and fuel use.

Scrappage and accelerated-retirement settings are partly represented in the Python workflow, but they are not currently written as routine T11 LEAP import variables. They should therefore be treated as testing or mirror-model inputs until the LEAP hand-off is deliberately extended.

Users should remember that even LEAP-editable variables interact with the stock-flow model. For example, changing sales shares affects future stock gradually through turnover. Changing mileage affects energy demand directly but does not change stock. Changing scrappage affects retirements and may affect replacement sales when it is enabled in a scenario workflow.

The general rule is:

- use the pre-LEAP workflow for difficult preparation, base-year calibration, and structural stock-flow assumptions; and

- use LEAP for official projection runs and controlled scenario changes.

The user will need to be careful not to change other variables in LEAP that are not intended to be scenario levers. For example, changing stock shares or survival assumptions directly in LEAP can have unintended consequences for the stock-flow pathway. The pre-LEAP workflow is designed to prepare those assumptions in a consistent way, so they should generally be left unchanged in LEAP unless the user understands the implications.

## 2.15	14. Python simulated outputs, LEAP, and official results

LEAP is the official projection platform. Once the road inputs are prepared and imported, LEAP calculates the road transport projection as part of the wider APERC energy system model.

The Python workflow supports LEAP by preparing inputs, reconciling the base year, generating import workbooks, and running optional QA checks. It can also run a mirror calculation to help users understand expected behaviour before or after LEAP is run.

The Python mirror and interface outputs should be called simulated outputs or QA outputs. They are useful because they show the likely effect of assumptions before the full LEAP model is run, and they help users understand whether inputs are plausible. However, the final result still depends on LEAP’s own stock-turnover calculations, scenario expressions, calculation order, and integration with the wider energy system.

This matters because road transport affects the rest of the energy system. Road fuel demand can affect electricity generation, refining, biofuels, hydrogen, oil product demand, imports, exports, and the final energy balance.

A useful way to interpret the workflow is:

Interface = review and update assumptions
Python workflow = prepare, reconcile, and simulate/check road inputs
LEAP = official projection and integration with the whole energy system
Dashboards = compare, diagnose, and explain results

## 2.16	15. Open methodology items for Codex / developer follow-up

The following items should be checked against the current code before this document is treated as final.

### 2.16.1	Scrappage implementation boundary

Module 4 stores explicit scrappage values as scrappage_for_leap, and Module 7 can use scrappage inputs in the Python mirror. The current Module 6 LEAP-ready table does not emit routine Scrappage rows, so this is not yet a standard LEAP hand-off lever. Before routine use, decide the exact LEAP variable rows, branch levels, and interface controls.

### 2.16.2	Future sales-share seeding status

The current workflow can read explicit future sales-share inputs, use projected Module 1 sales-share rows, or fall back to base-year-flat trajectories. When previous Outlook trajectories are available, Module 5 re-anchors non-ICE drive trajectories to the new base-year shares while preserving the provided shape; ICE is the residual. If the shape-preserving method would make ICE negative, Module 5 switches that vehicle/scenario group to linear interpolation and records method flags.

### 2.16.3	Interface visibility status

The current static contract in road_model_inputs_interface/back-end/data/road_model/config/road_module1_static_contract.csv marks all rows as shown in the base interface. In the projected editing view, projected Sales Share rows are the exception: they are present in the hand-off contract but marked hidden, so they can seed the model without being part of the routine projected-assumption editing workflow.

# 3	Road transport model workflow guide

Purpose note
This document is the implementation-oriented workflow guide for the road transport model in leap_road_model. It is intended for the developer or supervisor of the system. For a shorter conceptual guide to the model, use road_transport_model_methodology.md, and for an overview of the model, see road_transport_model_overview.md.

This guide describes the Python-side workflow needed to prepare the road transport model before it is passed into LEAP.

End-to-end road model workflow

Primary reference for the full end-to-end workflow. Some implementation detail is not shown.

More simplified illustration of the modelling workflow.

The road model is a detailed model because it uses a stock-flow structure: vehicle stock, sales, retirements, mileage, efficiency, and fuel allocation all interact to produce energy use. Existing transport documentation describes this as a sales-based model where ownership assumptions, sales shares, survival curves, efficiency, and mileage combine to produce annual energy use in LEAP.

## 3.1	Contents

- Implementation status (current repo)

- Workflow runtime configuration

- Module 1 data-source contract (no hard-coded data)

- Overall Python / LEAP Split

- Input Format

- Road Branch Scope

- Module Sequence

- Module 1 - Road Input Data and Defaults

- Module 2 - Base-Year Road Structure

- Module 3 - Stock Target Projection

- Module 4 - Sales, Survival, Vintage, and Turnover

- Module 5 - Vehicle Sales Share Preparation

- Module 6 - LEAP Handoff, Reconciliation, and Device Shares

- Module 7 - Optional Python Mirror and Post-LEAP Validation

- T-Table Lineage and Output Inventory

- Validation Requirements

- Main Files

- Relationship to Other Documents

## 3.2	Implementation status

The road workflow now uses the Module 1 package from road_model_inputs_interface as the main input source. This package is loaded before Modules 2–6 and provides the base-year assumptions, projection settings, and reconciliation parameters needed by the road model.

New work should use the canonical long CSV package generated by road_model_inputs_interface. Older wide-format files in input_data/module1_defaults/ are still supported for compatibility during migration, but they are no longer the preferred input source.

The Module 1 package provides, at minimum:

- base-year stock, mileage, fuel economy, and sales-share inputs;

- survival and vintage profiles;

- passenger saturation and vehicle-equivalent weight assumptions;

- vehicle-type Stock Share rows;

- passenger stock growth and freight GDP-elasticity adjustment settings;

- PHEV electric utilisation assumptions;

- reconciliation weights and scalar bounds for Module 6.

The exact interface row contract is maintained in road_model_inputs_interface, not in this repo. The road workflow should load the generated package rather than recreating or hard-coding those rows.

For normal runs, use the road_model_inputs_interface package builder. The local scripts/generate_module1_defaults.py script should only be used as a compatibility or backfill helper.

At runtime, the workflow also checks that population, GDP, and ESTO energy inputs have the expected columns and cover the required projection years. This helps catch input problems early before they cause unclear errors later in the workflow.

## 3.3	Workflow runtime configuration

Runtime settings for codebase/road_workflow.py are managed in codebase/config/workflow_defaults.yaml.

This file controls how the workflow runs, not the model assumptions themselves. It can set things such as the economy/scenario run scope, base and final years, Module 1 package location, whether outputs and visualisations are saved, which modules are run or skipped, and selected diagnostic settings.

Model input values should still come from the generated Module 1 package. This includes base-year stock, mileage, fuel economy, survival curves, PHEV utilisation, projection adjustment factors, reconciliation weights, and scalar bounds. These should not be treated as fallback values in the runtime configuration file.

Runtime settings are applied in this order:

- built-in workflow defaults;

- settings in codebase/config/workflow_defaults.yaml, or another file passed with --workflow-config;

- explicit command-line options or run_for_economy(..., **config_overrides) values.

Common command-line examples:

python codebase\road_workflow.py 15_PHL --no-vis --module1-defaults-dir ..\road_model_inputs_interface\back-end\outputs\road_module1_defaults --module1-defaults-version v2026_06_05_road_module1_sources
python codebase\road_workflow.py 20_USA --skip-m7 --no-save-csv-outputs
python codebase\road_workflow.py 20_USA --module6-match-tolerance 0.02

## 3.4	Module 1 data-source contract

Module 1 input values should not be hard-coded in leap_road_model. The authoritative data sources are maintained in road_model_inputs_interface, mainly under:

road_model_inputs_interface/back-end/data/road_model/

This includes values such as reconciliation factors, PHEV utilisation, passenger saturation, vehicle-equivalent weights, passenger stock growth adjustments, freight GDP-elasticity adjustments, and workbook defaults.

The leap_road_model repo should consume the generated Module 1 package from those sources. It should not contain separate hard-coded copies of the same datasets, because that would make the two repos harder to keep consistent.

## 3.5	Overall Python / LEAP Split

Python prepares the road transport input package that LEAP needs. LEAP remains the official Outlook projection platform.

Python is responsible for:

- loading and standardising Module 1 inputs;

- preparing base-year road branch data;

- projecting target stock envelopes for passenger and freight road;

- preparing sales, survival, vintage, and turnover inputs;

- seeding base-year and future sales shares;

- reconciling base-year fuel energy to ESTO; and

- writing LEAP-ready tables and diagnostics.

LEAP is responsible for:

- running the official stock-turnover projection;

- storing researcher-entered future sales shares and scenario assumptions;

- applying future mileage, fuel economy, and scrappage assumptions; and

- producing final Outlook outputs.

Any Python calculation after the LEAP handoff should be labelled as QA, diagnostics, or a mirror model. It is not the official projection unless the workflow explicitly says so. However, the Python mirror can be a useful tool for checking LEAP results and testing alternative assumptions without needing to run LEAP.

## 3.6	Input Format

The target workflow consumes the canonical long Module 1 package contract from road_model_inputs_interface. The current adapter may still accept the older LEAP-style wide CSV as a compatibility artifact.

Canonical long package columns are:

- Economy

- Scenario

- Branch Path

- Variable

- Year

- Value

- Scale

- Units

- source/comment metadata where available

Scale is optional for backward compatibility. When present, numeric LEAP-style labels such as Thousand, Thousands, Million, Millions, Billion, and Billions are applied as multipliers before values enter the model. For example, a Stock value of 1.25 with Scale = Millions is loaded internally as 1,250,000 devices. % is preserved as a LEAP/display scale for share rows and is not converted to a fraction by the generic input parser.

Profile rows stay in the same CSV. They encode their profile index in the branch path, for example Age Profile\5, rather than using a mostly blank extra column. Current profiles are treated as global road-model profiles unless a later package adds more specific profile branch paths.

Generated per-economy filenames use stable underscore economy codes and overwrite in place:

road_module1_values_<ECONOMY>.csv
road_module1_values_20_USA.csv

The adapter converts those rows into the internal LEAP-style shape used by the rest of the workflow. Legacy package files may still provide:

- Branch Path

- Variable

- Scenario

- Region

- at least one year column, usually 2022

Useful optional legacy columns are:

- Scale

- Units

- Per...

- source and review metadata such as input_source, source_type, notes, and review_flag

The runtime parser ignores unrelated metadata columns, but the diagnostics should preserve useful source flags where possible.

## 3.7	Road Branch Scope

The road branch hierarchy is:

Demand
  Passenger road
    LPVs                                 [sizes: small, medium, large]
      ICE <size>
        Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG
        Biogasoline, Biodiesel, Biogas, Efuel
      HEV <size>
        Motor gasoline, Gas and diesel oil, Biogasoline, Biodiesel, Efuel
      EREV <size>
        Electricity, Motor gasoline, Biogasoline, Efuel
      PHEV <size>
        Electricity, Motor gasoline, Biogasoline, Efuel
      BEV <size>
        Electricity
      FCEV <size>
        Hydrogen
    Motorcycles
      ICE
        Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG
        Biogasoline, Biodiesel, Biogas, Efuel
      BEV
        Electricity
      FCEV
        Hydrogen
    Buses
      ICE
        Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG
        Biogasoline, Biodiesel, Biogas, Efuel
      BEV
        Electricity
      FCEV
        Hydrogen
  Freight road
    Trucks                               [sizes: medium, heavy]
      ICE <size>
        Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG
        Biogasoline, Biodiesel, Biogas, Efuel
      BEV <size>
        Electricity
      FCEV <size>
        Hydrogen
    LCVs
      ICE
        Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG
        Biogasoline, Biodiesel, Biogas, Efuel
      PHEV
        Electricity, Motor gasoline, Biogasoline, Efuel
      BEV
        Electricity
      FCEV
        Hydrogen

The intention is to provide detail where it is useful, not to make every branch equally detailed. LPVs receive the most detail because they are usually the largest part of the fleet, have better data, and are where many technology-transition assumptions are most policy-relevant. Smaller or less data-rich branches are kept simpler where extra detail would add work without improving the result. At the same time, the model still needs enough detail to represent the different needs of APEC economies. Reducing detail is useful for keeping the model understandable and manageable, but it has to be weighed against the need to represent economies with different fuels, technologies, and vehicle structures.

This hierarchy matters because variables are defined at different levels, which is a result of model design decisions, which was sometimes influenced by the way LEAP works.

This means a change at one level can affect many branches below it. For example, changing BEV sales share under LPVs affects future BEV stock, electricity demand, ICE stock share over time, and liquid fuel demand. Changing mileage at a fuel branch affects energy use for that branch, but does not directly change stock or sales.

Current scope rules (important when comparing to the 9th edition and earlier 10th edition LEAP models):

- HEV and EREV are LPV-only.

- Truck PHEV is out of scope.

- LPVs use small, medium, and large size labels.

- Trucks use medium and heavy size labels where truck-size splits are needed.

- Fuel Economy is the canonical Module 1 efficiency variable. Final On-Road Fuel Economy can be accepted only as a legacy input alias.

The changes were generally intended to The same vehicle/drive/size matrix should be used by Module 1 validation and Module 2 branch generation. Module 1 rows outside this matrix should be rejected or explicitly recategorized before export; Module 2 should not create branches outside this matrix during skeleton generation.

## 3.8	Why some road preparation happens before LEAP

LEAP remains the official projection platform, and many important interactions still happen inside LEAP. The reason part of the road model is prepared before LEAP is not that LEAP cannot represent stock-flow behaviour. The reason is that several of the road model inputs are difficult to prepare, calibrate, and review directly inside LEAP.

The main pre-LEAP preparation tasks are:

- the overall road stock and sales trajectory;

- survival and vintage profile assumptions;

- PHEV utilisation shares;

- base-year stock, mileage, and fuel economy estimates; and

- base-year reconciliation to ESTO road fuel totals.

These tasks involve linked calculations, source-data cleaning, judgement about uncertain inputs, and checks against historical energy balances. Preparing them outside LEAP simplifies the modelling process because researchers can review the main assumptions in a focused interface while Python prepares the dependent LEAP inputs consistently.

This should not be understood as a separation where Python does all interactions and LEAP simply stores the result. Some interactions occur before LEAP because they are easier to prepare there, while others occur in LEAP during the official stock-turnover projection. The pre-LEAP workflow is therefore a preparation and QA layer around LEAP, not a replacement for LEAP.

# 4	Module Sequence

codebase/road_workflow.py runs Modules 2 to 6 after loading a pre-generated Module 1 package. Module 7 is available as an optional QA mirror and is called separately.

Module 1 defaults package
  -> Module 2 base-year branch table
  -> Module 3 stock targets
  -> Module 4 sales, survival, vintage, turnover
  -> Module 5 sales shares
  -> Module 6 LEAP handoff and reconciliation
  -> optional Module 7 mirror and post-LEAP validation

## 4.1	Module 1 - Road Input Data and Defaults

### 4.1.1	Purpose

Module 1 gathers, standardises, and documents road model inputs before any stock projection, turnover, fuel allocation, or LEAP preparation is done.

The current orchestrator does not run raw Module 1 source processing directly. It loads a generated Module 1 package. During migration this can be a legacy package from input_data/module1_defaults/; the target package is the long CSV from road_model_inputs_interface.

The interface-side row contract is ../road_model_inputs_interface/back-end/data/road_model/config/road_module1_static_contract.csv. It is the operator-maintained list of expected static package rows and interface visibility flags; the model should load the generated package rather than reconstructing that contract itself.

Researchers who want to run the road workflow without starting the website can use scripts/offline_workflow.py. It reads pre-generated Module 1 outputs from the sibling road_model_inputs_interface/back-end/outputs/road_module1_defaults/ directory when available, falls back to legacy input_data/module1_defaults/, and then calls run_for_economy() for one or more economies.

### 4.1.2	Responsibilities

Module 1 provides:

- base-year stock, mileage, fuel economy, and sales-share rows;

- survival curves and vintage profiles;

- passenger saturation assumptions;

- vehicle-equivalent weights;

- passenger stock growth-rate adjustment;

- freight GDP-elasticity adjustment;

- PHEV electric utilisation assumptions;

- reconciliation weights and scalar bounds where available;

- source and comment metadata for defaulted or researcher-edited values.

### 4.1.3	Researcher Input Tool

The researcher input process should guide users through missing data, show defaults, and allow economy-specific overrides. It should not require researchers to know every value before the workflow can run.

The tool should collect or expose:

- base-year stock by vehicle type, drive, and size where relevant;

- base-year mileage;

- base-year fuel economy;

- base-year sales shares where known;

- survival and vintage assumptions;

- saturation and vehicle-equivalent weight assumptions;

- passenger stock growth-rate adjustment;

- freight GDP-elasticity adjustment;

- PHEV utilisation assumptions;

- notes explaining researcher overrides.

### 4.1.4	Outputs

The generated Module 1 package should include:

- one canonical long CSV for each economy;

- a manifest describing generation date, base year, source files, and scripts;

- any validation report generated during packaging; and

- source/comment metadata needed for diagnostics.

## 4.2	Module 2 - Base-Year Road Structure

### 4.2.1	Purpose

Module 2 parses Module 1 rows and produces a base-year road branch table that later modules can use consistently.

### 4.2.2	Responsibilities

Module 2 does the following:

- build the full road branch skeleton from vehicle_mappings.yaml and fuel_mappings.yaml;

- join parsed Module 1 base-year stock, mileage, and efficiency rows onto that skeleton;

- keep passenger and freight road separate;

- preserve source flags and granularity diagnostics for the joined base-year values;

- reject zero mileage or zero efficiency placeholders before reconciliation; and

- build a tidy branch table used by Modules 3 to 6.

Branch-path parsing, variable normalisation, and scale multipliers happen before Module 2 in the Module 1 adapter/parser. Module 2 also has pragmatic fill rules: when only vehicle-type-level stock is available, it distributes stock across the matching drive/size branches and records vehicle_type_equal_split; when only vehicle-type-level mileage or efficiency is available, it broadcasts those values to matching branches and records vehicle_type_level_broadcast.

### 4.2.3	Important Distinction

Module 2 prepares the branch structure and base-year inputs. It does not perform final fuel reconciliation. ESTO matching and Device Share finalisation happen in Module 6.

### 4.2.4	Outputs

Module 2 produces T4_base_year_branches, the common base-year branch table used by Modules 3 to 6.

## 4.3	Module 3 - Stock Target Projection

### 4.3.1	Purpose

Module 3 creates target stock envelopes for passenger and freight road. Target stock is calculated internally; it is not treated as a fixed external input.

Vehicle-type stock splits come from Module 1 Stock Share rows at the five vehicle-type branches: Demand\Passenger road\LPVs, Demand\Passenger road\Motorcycles, Demand\Passenger road\Buses, Demand\Freight road\Trucks, and Demand\Freight road\LCVs. Values are LEAP-style percentages in the Module 1 package. The workflow converts them to fractions, interpolates from the base year to supplied target years, and holds the last target constant.

Passenger Stock Share (LPVs, Motorcycles, Buses) is researcher-adjustable. Module 1 provides base-year shares derived from base-year Stock rows, plus default anchor values at 2040 and 2060 seeded to the same base-year share. Researchers may edit the 2040/2060 anchors to define a trajectory; if left unchanged, the model holds the base-year split flat. The workflow converts passenger physical shares to LPV-equivalent capacity shares using Module 1 vehicle-equivalent weights before calling Module 3.

Freight Stock Share (Trucks, LCVs) is not researcher-adjustable. The truck/LCV split shows no meaningful projected trend and is held flat at the base-year proportions. The 2040 and 2060 anchor values are seeded equal to the base year by design; researchers should leave these unchanged.

### 4.3.2	Passenger Stock

Passenger stock is projected using a GDP-per-capita motorisation envelope. The envelope is defined in vehicle-equivalent terms so that the ownership curve remains comparable when the vehicle mix shifts between LPVs, motorcycles, and buses. GDP per capita drives structural growth in ownership for economies that have not reached saturation; the growth effect is damped as vehicle-equivalent ownership approaches the saturation level.

#### 4.3.2.1	Vehicle-equivalent ownership

Physical stocks are converted to a single aggregate using Module 1 Vehicle Equivalent Weight values (sourced from apec_vehicle_equivalent_weights.csv). Default weights:

| Vehicle type | Default weight |
| --- | --- |
| LPVs | 1.0 (reference unit) |
| Motorcycles | 0.8 |
| Buses | 20.0 |

These are config-driven, not hard-coded. One bus is treated as equivalent to 20 LPVs of ownership demand; ignoring this would produce a meaningless aggregate when bus-heavy economies are compared with car-dominated ones.

Base-year motorisation level:

M_base = sum(base_stock[vt] Ã— weight[vt]) / population_base

in units of LPV-equivalents per capita.

#### 4.3.2.2	Saturation level

At runtime, M_sat is resolved in priority order:

- Passenger saturation value loaded from the generated Module 1 package.

- Fallback: M_sat = M_base Ã— 3.0.

Module 1 may have sourced its saturation value from researcher edits or a regional/default source package, but the current workflow passes the loaded Module 1 value into Module 3 as a researcher saturation override. That means T5 saturation_source_flag distinguishes researcher versus fallback in routine runs; finer provenance should be checked in the Module 1 source metadata rather than inferred from the T5 flag.

#### 4.3.2.3	Estimating passenger income elasticity

Passenger growth uses historical passenger road energy as the activity proxy and compares it with historical GDP-per-capita growth. Both growth rates are endpoint compound rates over the filtered lookback window, matching the freight elasticity style:

passenger_energy_growth = (E_end / E_start) ^ (1 / n) - 1
gdp_per_capita_growth   = (GDPpc_end / GDPpc_start) ^ (1 / n) - 1
income_elasticity       = passenger_energy_growth / gdp_per_capita_growth

COVID years 2020-2022 are excluded before selecting the first and last usable observations. The lookback window and excluded years are config-driven (lookback_window_years = 10, covid_exclude_years = [2020, 2021, 2022]). If either series has fewer than two usable observations, if the first usable value is not positive, or if GDP-per-capita growth is effectively zero, the method uses the default passenger income elasticity (0.8).

The raw estimated elasticity is clamped to [0.0, 2.0]. The bounded value is then multiplied by the Module 1 Passenger Stock Growth Rate Adjustment and checked against the same bounds again. The current APEC-wide default adjustment is 1.2, meaning the estimated income response is increased by 20 percent unless an economy-specific or researcher-edited value overrides it. A value of 1.0 keeps the estimated elasticity unchanged.

#### 4.3.2.4	GDP-per-capita projection with saturation damping

For each projection year, the model applies GDP-per-capita growth to the previous year’s vehicle-equivalent ownership level:

income_growth_factor = (GDPpc[year] / GDPpc[previous_year]) ^ income_elasticity
income_growth_rate   = income_growth_factor - 1
remaining_fraction   = max(0, 1 - M[previous_year] / M_sat)
M[year]              = M[previous_year] * (1 + income_growth_rate * remaining_fraction)

M[year] is capped at M_sat and not allowed to go below zero. This keeps the projection responsive to future GDP-per-capita trends while ensuring that growth fades as ownership approaches saturation.

#### 4.3.2.5	Already-saturated economies

If M_base >= 0.95 x M_sat, the economy is treated as already saturated: passenger income elasticity is set to 0.0 and the is_saturated flag is set. Ownership remains flat; stock changes only with population.

#### 4.3.2.6	Weight calibration

When passenger_saturation_reached = True, Module 3 can calibrate vehicle-equivalent weights for Motorcycles and Buses so that the base-year weighted stock equals the saturation target. Calibration solves a constrained minimisation that stays within per-type bounds and minimises deviation from the Module 1 default weights. The original and adjusted weights are both recorded in T5.

#### 4.3.2.7	Allocation to vehicle types

After projecting the aggregate motorisation envelope M(year):

total_weighted(year) = M(year) Ã— population(year)
target_stock(vt, year) = total_weighted(year) Ã— capacity_share(vt) / weight(vt)

where capacity_share(vt) = base_stock(vt) Ã— weight(vt) / sum(base_stock Ã— weight).

#### 4.3.2.8	Post-reconciliation stock target adjustment

After Module 6 reconciles base-year stock, mileage, and efficiency to ESTO fuel totals, the workflow may call road_workflow.py::build_post_reconciliation_stock_targets(). For passenger rows, this re-anchors the base year to reconciled stock but preserves the original final-year physical stock target. The difference between the pre-reconciliation base-year stock and reconciled base-year stock fades linearly to zero by final_year. T5 records the original target, reconciled base stock, base adjustment, and method string preserve_final_target_linear_base_adjustment.

### 4.3.3	Freight Stock

Freight stock is projected with a bounded GDP-elasticity method using estimate_freight_elasticity and project_freight_stocks in module3_stock_targets.py.

#### 4.3.3.1	Elasticity estimation

Freight uses historical freight road energy growth as an activity proxy, then compares it with GDP growth. Both growth rates are endpoint compound rates over the filtered 10-year lookback window, not averages of annual log changes:

freight_energy_growth = (E_end / E_start) ^ (1 / n) âˆ’ 1
gdp_growth            = (GDP_end / GDP_start) ^ (1 / n) âˆ’ 1
elasticity            = freight_energy_growth / gdp_growth

COVID years are excluded before selecting the first and last usable observations. Here n is the number of intervals between the remaining observations. If either series has fewer than two usable observations, if the first usable value is not positive, or if GDP growth is effectively zero, the method uses the default elasticity instead of estimating one from the ratio.

The raw estimated elasticity is first clamped to [0.0, 2.0] inside estimate_freight_elasticity(). That bounded value is then multiplied by Module 1 Freight GDP Elasticity Adjustment and checked against the same bounds again. If GDP growth is near zero or data are insufficient, the default elasticity 0.8 is used. The data source flag (estimated, estimated_adjusted, default, or override) and a short note are carried into T5.

Historical GDP rows are required for this estimate. If the macro input starts at the base year only, the lookback window cannot estimate GDP growth, especially because 2020-2022 are excluded as COVID years. The intended fix is to add historical GDP data to the macro source, not to add code that hides the missing-data warning.

#### 4.3.3.2	Stock projection

total_base = Trucks_base + LCVs_base
total(year) = total_base Ã— (GDP(year) / GDP_base) ^ elasticity
target_stock(vt, year) = total(year) Ã— physical_share(vt)

where physical_share(vt) = base_stock(vt) / total_base and shares are held flat at base-year proportions (see the Stock Share discussion above).

A researcher-supplied freight_total override replaces the estimated elasticity; the override is recorded in diagnostics.

#### 4.3.3.3	Freight physical-count method and weighted-index interpretation

The current freight stock projection uses physical vehicle counts rather than a weighted freight capacity index. This is acceptable under the current modelling design because the truck/LCV split is held flat at base-year proportions. If the split is fixed, projecting total physical freight stock and then allocating it to trucks and LCVs gives the same vehicle-count result as projecting a weighted freight index and then converting it back to physical stocks.

A weighted freight index would become more important if researchers were later allowed to change the truck/LCV mix over time. In that case, one truck and one LCV should not be treated as equal units, because they represent different freight-carrying capacity. If time-varying truck/LCV shares are introduced in a future model version, the freight projection method should be reviewed so that the aggregate freight stock pathway is defined in capacity-equivalent terms rather than simple physical vehicle counts.

#### 4.3.3.4	Post-reconciliation stock target adjustment

When post-reconciliation stock target adjustment is enabled, freight rows are re-anchored to reconciled base-year stock differently from passenger rows. Freight preserves the original growth index from Module 3:

growth_factor(year) = pre_reconciliation_target_stock(year) / pre_reconciliation_target_stock(base_year)
post_reconciliation_target_stock(year) = reconciled_base_stock Ã— growth_factor(year)

This keeps the GDP-elasticity growth shape unchanged while moving the physical stock level to the reconciled base. T5 records the method string preserve_growth_index_from_reconciled_base.

### 4.3.4	Diagnostics

T5 carries the following per-row diagnostic columns for review:

Passenger rows: motorisation_level, saturation_level, passenger_income_elasticity_used, passenger_raw_income_elasticity, passenger_income_elasticity_clamped, passenger_energy_growth_rate, passenger_gdp_per_capita_growth_rate, passenger_income_elasticity_data_source, passenger_stock_growth_rate_adjustment, is_saturated, saturation_source_flag, original_vehicle_equivalent_weight, adjusted_vehicle_equivalent_weight, weight_calibration_applied. Legacy k_raw, k_used, and k_clamped columns may still appear for compatibility but are not used by the current passenger projection.

Freight rows: gdp_elasticity_used, freight_raw_elasticity, freight_elasticity_clamped, freight_energy_growth_rate, freight_gdp_growth_rate, freight_elasticity_adjustment, freight_elasticity_data_source, freight_elasticity_note.

### 4.3.5	Outputs

Module 3 produces T5_stock_targets with columns for economy, scenario, year, transport_type, vehicle_type, target_stock, and all diagnostic columns above.

## 4.4	Module 4 - Sales, Survival, Vintage, and Turnover

### 4.4.1	Purpose

Module 4 converts the stock targets from Module 3 into the sales, retirements, lifecycle profiles, and scrappage assumptions that LEAP needs for its stock-turnover calculation.

Module 3 says how many vehicles should exist in each future year. Module 4 explains how the fleet gets there: how many existing vehicles survive, how many retire naturally, whether any additional scrappage is applied, and how many new vehicles must be sold to reach the target stock.

### 4.4.2	Core stock-flow logic

For each vehicle bucket, Module 4 applies the stock-flow identity:

ending stock = surviving prior stock + new sales - additional retirements

In practice, this means the model:

- applies survival curves to estimate how much of the existing fleet remains;

- uses the base-year vintage profile to represent the starting age structure of the fleet;

- calculates natural retirements from the survival assumptions;

- applies any additional scrappage assumptions where relevant;

- calculates the sales required to reach the target stock;

- prevents negative sales; and

- records stock-accounting diagnostics for each year and vehicle bucket.

This step is important because stock targets alone do not tell us how quickly the vehicle fleet changes. A stock pathway can be reached through slow turnover, rapid replacement, or policy-driven scrappage, and each pathway has different implications for technology uptake and energy use.

### 4.4.3	Why survival and vintage profiles matter

A survival curve describes the probability that a vehicle is still operating at each age. A long-lived survival curve keeps vehicles in the fleet for longer, reducing near-term replacement sales. A shorter survival curve increases retirements and creates more opportunities for new technologies to enter the fleet.

A vintage profile describes the age distribution of the fleet in the base year. Two economies may have the same number of vehicles, but if one fleet is older, it may have much higher near-term retirements. This means that the same stock target can produce very different sales pathways depending on the starting fleet age structure.

Together, the survival curve and vintage profile determine the speed of turnover. This affects the speed at which BEVs, FCEVs, PHEVs, or more efficient ICE vehicles can replace the existing fleet.

### 4.4.4	Profile checking and lifecycle calibration

Survival and vintage profiles are not imported passively. Module 4 checks and prepares them before they are used in the stock-flow calculation.

The workflow should:

- normalise survival curves and vintage profiles;

- align profiles to the same age range;

- preserve the broad shape of researcher-provided profiles where possible;

- apply lifecycle calibration factors where needed;

- keep turnover rates within reasonable bounds;

- apply any base-year fleet age shift before the turnover calculation; and

- record the resulting lifecycle assumptions for review and LEAP export.

The objective is to keep the lifecycle assumptions close to the user-provided or default profiles while still producing a coherent stock-flow pathway. In other words, the model should respect the intended survival and vintage shape, but it must also ensure that the resulting sales, retirements, and stocks remain consistent with the projected stock targets.

### 4.4.5	Handling cases where surviving stock exceeds the target

Sometimes the vehicles that survive from previous years are already greater than the target stock. This can happen when stock is flat or declining but vehicles have long assumed lifetimes.

In this case, the model does not create negative sales. Instead, it scales down the surviving cohorts to match the target stock, records stock_above_target = True, and stores the scale_factor_applied. This makes the adjustment visible in diagnostics rather than hiding it inside the sales calculation.

This is especially important for economies or vehicle types where the projected stock pathway declines, because the model needs to distinguish between normal replacement sales and situations where the existing fleet would otherwise remain larger than the target.

### 4.4.6	Additional scrappage

Additional scrappage represents early retirement beyond the normal survival curve. It can be used to test policies that retire older, inefficient, or high-emitting vehicles faster than normal fleet turnover would imply.

Scrappage should be treated carefully because it affects several parts of the model at once. It changes retirements, sales, stock accounting, technology uptake, and future energy use. For example, scrapping ICE vehicles faster only reduces long-term energy use if replacement sales are directed toward more efficient or lower-emission technologies.

Temporary scrappage policies should be represented as explicit year-specific scrappage assumptions rather than being hidden inside a permanent survival-curve change, unless the policy is intended to represent a lasting structural change in vehicle lifetimes.

### 4.4.7	Relationship to LEAP

Module 4 prepares the lifecycle and turnover inputs that LEAP needs, but LEAP remains the official projection platform. The Python workflow calculates a coherent set of stock, sales, retirement, survival, vintage, and scrappage inputs so that LEAP can apply its own stock-turnover logic using consistent assumptions.

Because LEAP may calculate stock turnover in a slightly different order or with different internal handling, Python-side turnover outputs should be treated as preparation and QA outputs unless explicitly stated otherwise. The main role of Module 4 is to produce LEAP-ready lifecycle inputs and to make the implied stock-flow pathway transparent before the model is imported into LEAP.

### 4.4.8	Outputs

Module 4 produces T6_sales_turnover, which includes:

- target stock;

- surviving stock;

- natural retirements;

- additional retirements;

- required sales;

- final stock after any cohort adjustment;

- stock_above_target and scale_factor_applied where surviving cohorts exceed the target stock;

- explicit scrappage_for_leap values where supplied; and

- stock-accounting diagnostics.

Module 4 also produces T6v_vintage_profiles, the age-profile table used by the lifecycle exporter. This table records the vintage shares, survival probabilities, and any age-shift marker needed for the LEAP lifecycle profile workbooks.

The workflow writes LEAP-compatible lifecycle profile workbooks from the final active T6v table under:

results/<economy>/lifecycle_profiles/

Survival profiles are exported as cumulative percent curves reconstructed from Module 4 annual survival probabilities. Vintage profiles are exported as percent age distributions that sum to 100. The output directory includes one workbook per vehicle/profile type, a lifecycle_profile_manifest.csv, and a downloadable <economy>_lifecycle_profiles.zip.

## 4.5	Module 5 - Vehicle Sales Share Preparation

### 4.5.1	Purpose

Module 5 prepares base-year sales shares and seeded future sales-share trajectories for the road model.

Sales shares determine how new vehicle sales are allocated across vehicle types, drive technologies, sizes, and, where relevant, fuel branches. They are one of the main ways that technology uptake enters the model.

For example, LPV sales shares can determine how new sales are divided between ICE, HEV, PHEV, BEV, and FCEV vehicles. Freight sales shares can determine how new truck or LCV sales are divided between ICE, BEV, FCEV, and other included technologies.

Sales shares affect the model gradually through stock turnover:

sales shares -> new sales by technology -> stock by technology -> energy by fuel

This delay is central to stock-flow modelling. A rapid increase in BEV sales does not immediately convert the whole fleet to BEVs. The effect grows over time as older vehicles retire and new BEVs accumulate in the stock.

Module 5 is relatively small compared with the stock projection, turnover, and reconciliation modules, but it is important because future sales shares strongly affect the technology pathway.

### 4.5.2	Base-year sales shares > TODO

Base-year sales shares initialise the stock-turnover model. They describe how new sales are distributed across the road branch structure in the base year.

Module 5 is still needed even when Module 1 provides consistent base-year Sales Share rows. The current implementation computes a base-year share table from T4 as a fallback, then applies Module 1 Sales Share rows as overrides. This keeps downstream tables on one normalised schema and gives Module 6 a complete sales-share trajectory table.

Module 5 also acts as a defensive validation step. It checks that sales shares are non-negative and sum correctly within each economy, scenario, year, transport type, and vehicle type. In routine runs these checks should already pass because Module 1 should have enforced the same contract, but keeping the checks here helps catch problems before the LEAP handoff.

### 4.5.3	Future sales-share seeding

Future sales shares are mainly intended to be edited by researchers in LEAP. LEAP remains the official projection platform for technology uptake and stock-turnover scenario results.

However, the Python workflow can seed future sales-share trajectories before the LEAP handoff. These seeded shares are a bridge assumption, not an official policy scenario. They provide a more realistic starting point than holding base-year sales shares flat, and they allow Module 7 to produce more useful simulated or QA outputs before LEAP is run.

In normal use, future sales shares are based on 9th Edition trajectories scaled to match the new base year. The Module 1 interface also has an option to edit future sales shares, but this is currently only exposed in the “Ultra” data-density mode and is unlikely to be used by most researchers. For most users, the expected workflow is therefore:

- Python seeds a reasonable future sales-share pathway.

- The pathway is imported into LEAP.

- Researchers review and edit future sales shares in LEAP as part of scenario development.

- LEAP produces the official projection results.

Pre-LEAP sales-share results should therefore be described as simulated outputs or QA outputs. They help users understand the likely effect of assumptions, but the official result depends on how LEAP applies its own stock-turnover calculations, scenario expressions, and calculation order.

### 4.5.4	Future sales-share methods

The current implementation can create T7f_future_shares using several methods. Method flags should make the source of each trajectory clear:

- shape_preserve: preserve the 9th Edition trajectory shape after anchoring to the new base year;

- hold_flat: no source trajectory exists, so hold the new base-year share constant;

- hold_at_base: the source trajectory is flat, so hold the new base-year share constant.

These flags are useful because users need to know whether a future technology pathway came from a previous Outlook trajectory, a flat fallback, or another assumption source.

### 4.5.5	Outputs

Module 5 produces:

- T7_sales_shares: base-year sales shares by road bucket;

- T7f_future_shares: seeded future sales-share trajectories where available;

- method/source flags showing how future sales shares were created; and

- fallback diagnostics for missing or flat sales-share trajectories.

## 4.6	Module 6 - LEAP Handoff, Reconciliation, and Device Shares

### 4.6.1	Purpose

Module 6 builds the final Python-side road package for LEAP and reconciles base-year energy to ESTO. It is the most complex module because fuel energy, stock, mileage, efficiency, and Device Shares all interact.

### 4.6.2	Handoff Package

Module 6 combines:

- Module 2 base-year branch data;

- Module 4 sales and lifecycle outputs;

- Module 5 sales shares;

- ESTO road fuel totals;

- Module 1 PHEV utilisation and reconciliation settings.

The LEAP-ready package preserves economy, scenario, year, transport type, vehicle type, drive type, size, fuel, variable, value, units, and source metadata.

### 4.6.3	Base energy formula

All energy calculations in Module 6 use:

energy_pj = stock × mileage / efficiency_km_per_gj / 1,000,000

where efficiency is distance per unit energy (km/GJ). Higher efficiency means lower energy use per vehicle.

### 4.6.4	Reconciliation Workflow

The nine-step workflow is implemented in run_module6 (codebase/modules/module6_reconciliation_and_leap_handoff.py). The file name intentionally covers both major responsibilities: base-year reconciliation and the LEAP hand-off package.

#### 4.6.4.1	Step 1 â€” Initial branch energy

Calculate energy_pj for every branch using base-year stock, mileage, and efficiency from Module 2.

#### 4.6.4.2	Step 2 â€” BEV and PHEV electricity reconciliation

Before touching liquid fuels, reconcile BEV and PHEV electricity to the ESTO road electricity total using the same scalar method as Steps 5â€“6. PHEV electricity and liquid fuel are calculated from the adjusted PHEV branches using the Module 1 PHEV electric utilisation rate, which is held fixed during reconciliation unless config explicitly allows it to move.

Note that EREVs are considered as PHEVs for reconciliation purposes because they have the same electric drive and fuel use characteristics as PHEVs. The difference is that EREVs have a larger battery and can run on electricity alone for longer distances, but they still use liquid fuel when the battery is depleted. Therefore, both PHEVs and EREVs contribute to the electricity total and are reconciled together before adjusting liquid fuel.

#### 4.6.4.3	Step 3 â€” PHEV liquid fuel subtraction

Remove PHEV liquid fuel from the relevant ESTO pools before normal fuel reconciliation:

remaining_esto_gasoline = ESTO_gasoline − PHEV_gasoline

PHEV and EREV liquid fuel is gasoline-family only for both passenger and freight branches: Motor gasoline, Biogasoline, and Efuel are allowed. Diesel and biodiesel are not allocated to PHEV/EREV branches. This prevents ICE reconciliation from absorbing gasoline-family energy that actually belongs to PHEVs/EREVs while keeping diesel out of the plug-in hybrid fleet.

#### 4.6.4.4	Step 4 â€” Allocate remaining ESTO fuel to eligible branches

For each fuel, the remaining ESTO total is allocated across eligible branches. Eligibility is driven by fuel_mappings.yaml (drive_fuel_eligibility). This creates a provisional allocated_branch_fuel_energy_pj for each branch before scalar adjustment.

Conventional liquid fuels use a vehicle-priority spillover rule before falling back to energy-share allocation:

- Diesel and biodiesel are allocated through the freight priority tiers first: freight ICE branches first, with Trucks and LCVs sharing the same tier by initial branch energy, then passenger vehicles only if freight liquid-fuel demand has already been saturated.

- Gasoline and biogasoline are allocated to passenger vehicles first, then any leftover spills to LCVs. They are not allocated to trucks in the road priority rule.

- Within each priority tier, allocation is proportional to initial branch energy.

- If the ESTO total exceeds the modeled energy capacity of all priority tiers, the final residual is assigned within the final allowed priority tier by initial-energy share so the whole fuel total remains represented in T8 without violating the reviewed vehicle order.

Biofuels follow the same priority families as their fossil counterparts: Biodiesel follows diesel tiers and Biogasoline follows gasoline tiers. The current model allocates the observed ESTO biofuel total as its own fuel stream to eligible branches, but should not be interpreted as a dedicated biofuel fleet.

Other ordinary fuels without a reviewed priority order use initial-energy-share allocation across eligible branches.

#### 4.6.4.5	Step 5 â€” Derive energy correction factor

For each branch:

ECF = allocated_branch_fuel_energy_pj / initial_branch_energy_pj

If a branch has zero initial energy but non-zero allocated ESTO energy (e.g. a hydrogen FCEV branch in an economy with no observed hydrogen use), the ECF is treated as zero and the branch scalars are clamped to their lower bounds rather than raising a division error.

Before this step, Module 6 can bootstrap eligible zero-stock fuel branches with a small seed stock when ESTO reports a positive fuel total and every eligible branch has zero initial energy. After scalar calculation, zero-stock rows can also inherit mileage and efficiency scalars from peer branches using a hierarchy of same drive type, same vehicle type and size, then same vehicle type.

#### 4.6.4.6	Step 6 â€” Adjust stock, mileage, and efficiency simultaneously

The ECF is split across the three variables using configurable Module 1 weights that are exposed through the interface and can be edited by researchers (defaults: stock=0.50, mileage=0.25, efficiency=0.25). The defaults put more weight on stock because stock is usually more visible and easier to review than mileage or efficiency.

stock_scalar      = ECF ^ 0.50
mileage_scalar    = ECF ^ 0.25
efficiency_scalar = ECF ^ −0.25     ← inverted: higher efficiency reduces energy

Because efficiency is km/GJ, increasing it lowers energy use. The negative exponent ensures the efficiency scalar moves in the direction that corrects the energy gap.

Each scalar is clamped to its configured per-scalar bounds from Module 1 (reconciliation_bound_lower/upper_stock/mileage/efficiency). Bounds are per-scalar by default, allowing stock wider movement than mileage or efficiency. Legacy single-tuple bounds are still accepted by the code for older packages and tests, but routine generated Module 1 packages should use per-component bounds.

Adjusted values:

adjusted_stock      = stock      × stock_scalar
adjusted_mileage    = mileage    × mileage_scalar
adjusted_efficiency = efficiency × efficiency_scalar

Whether each scalar stayed within bounds is recorded in T9 and T12.

#### 4.6.4.7	Step 7 â€” Recalculate final branch fuel energy

final_energy_pj = adjusted_stock × adjusted_mileage / adjusted_efficiency / 1,000,000

This should match allocated_branch_fuel_energy_pj within tolerance. If scalars were clamped, a residual gap will remain and is reported in T12.

#### 4.6.4.8	Step 8 â€” Calculate implied vehicles and Device Shares

energy_per_vehicle = adjusted_mileage / adjusted_efficiency / 1,000,000
implied_vehicles   = final_energy_pj / energy_per_vehicle
Device Share       = implied_vehicles / adjusted_total_vehicles_in_branch

Device Shares must be calculated after reconciliation because they depend on the final reconciled stock, mileage, efficiency, and fuel allocation. Single-fuel drive types (e.g. BEV) always have Device Share = 1.0.

#### 4.6.4.9	Step 9 â€” Validate

T12 diagnostics check:

- final fuel energy matches ESTO totals (after adding PHEV liquid back) within tolerance;

- all scalars stayed within bounds;

- Device Shares sum to 1 within each parent branch;

- no negative values or impossible fuel/drive combinations.

### 4.6.5	Fuel Allocation Rules

Fuel eligibility is config-driven (fuel_mappings.yaml), not hard-coded. Key rules:

| Fuel | Eligible drives | Notes |
| --- | --- | --- |
| Motor gasoline | ICE, HEV, PHEV, EREV | Biogasoline follows the same gasoline-family branches |
| Gas and diesel oil | ICE, HEV | Biodiesel follows the same diesel-family branches; freight-preferred allocation where ESTO diesel is freight-dominated |
| LPG | ICE only | Separate fuel from natural gas |
| Natural gas | ICE only | Separate fuel from LPG; biogas follows the same branches |
| Electricity | BEV, PHEV, EREV | Handled in Step 2 before normal reconciliation |
| Hydrogen | FCEV | Not expected in most base years; clamped to lower bound if no ESTO hydrogen observed |
| E-fuels | ICE, HEV, PHEV, EREV | Not expected in most base years; follows ordinary liquid-fuel ICE-style branches unless a narrower reviewed rule exists |
| Ammonia | Not assigned to road unless a reviewed branch rule exists and the code is adjusted where necessary | â€” |

Any fuel present in ESTO but with no valid branch is flagged in T12.

### 4.6.6	PHEV Treatment

PHEV and EREV branches carry two fuel streams: electricity and gasoline-family liquid fuel. EREV is treated like a more efficient PHEV. The electric utilisation rate (fraction of km driven on electricity) is a Module 1 input from apec_phev_utilisation_rates.csv and is held fixed during reconciliation. it is determined separately for passenger and freight branches because the utilisation rate can differ between the two.

After BEV/PHEV electricity is reconciled in Step 2, PHEV liquid fuel is computed and removed from the ESTO pools before ICE reconciliation. This preserves the ESTO fuel balance:

final_gasoline = reconciled_non_phev_gasoline + phev_gasoline
final_biogasoline = reconciled_non_phev_biogasoline + phev_biogasoline
final_efuel = reconciled_non_phev_efuel + phev_efuel
final_diesel   = reconciled_non_phev_diesel

build_phev_utilisation_diagnostics() back-calculates the electric driving share from reconciled PHEV/EREV electric and liquid energy, compares it with the Module 1 input rate, and writes the result to T12_phev. Rows outside the configured tolerance are flagged as below_range or above_range, which helps identify fuel allocation or reconciliation problems that distort the PHEV electric/liquid split.

### 4.6.7	Outputs

Module 6 produces:

- T8: fuel allocation table (provisional allocated branch fuel energy);

- T9: reconciliation scalar table and reconciled branch values (stock, mileage, efficiency, scalars, within-bounds flags);

- T10: Device Share table;

- T11: LEAP-ready output table;

- T12: reconciliation diagnostics (ECF, residual gaps, validation status);

- T12_phev: PHEV utilisation diagnostics.

T11 is written at LEAP-compatible variable levels:

- Stock: transport-type and vehicle-type branches;

- Sales: transport-type branches;

- Mileage, Fuel Economy, Device Share: fuel-level branches;

- Sales Share, Stock Share: vehicle-type rows and drive/technology rows.

When a reference LEAP export is available, the final Excel workbook is written through codebase/adapters/leap_import_writer.py, which merges BranchID, VariableID, ScenarioID, and RegionID and returns structured warnings for unmatched rows. The workflow resolves a reference LEAP export via _default_leap_reference_path() and uses config/road model leap export.xlsx as its primary source. This file is a real export from a fully-configured road transport LEAP model and is the authoritative source of BranchID, VariableID, ScenarioID, and RegionID for every branch × variable × scenario combination. The ID columns must match LEAP’s own internal IDs or the import will fail. The file must be regenerated from LEAP whenever the road transport branch structure is changed (branches added, renamed, or removed). All APEC economy LEAP models share the same “clean slate” road transport branch structure, so one export file covers all economies. If the repo-local reference file is missing, the workflow can fall back only to the reviewed full-model export under leap_utilities; if no candidate is found, the strict writer is bypassed and LEAP cannot import the output. The LEAP sheet keeps LEAP’s import column structure: ID columns, Branch Path, Variable, Scenario, Region, Scale, Units, Per..., Expression, a blank spacer column, and Level 1 through Level 8.... The Level columns are derived directly from Branch Path by splitting on \, with unused levels left blank.

Lifecycle profile files are written separately by codebase/adapters/lifecycle_profile_exporter.py because LEAP lifecycle profiles use a different workbook layout from the main branch-variable import workbook.

A second-run calibration csv output is available, which exports a Module 1-compatible CSV containing reconciled base-year stock, mileage, and fuel economy values, as well as all other previous inputs, so researchers can import those values into the interface and rerun the workflow with near-flat reconciliation adjustments. Other uses for this output may be as a simplified record of the LEAP inputs which can be sent to collaborators or used for future reference.

## 4.7	Module 7 - Optional Python Mirror and Post-LEAP Validation

### 4.7.1	Purpose

Module 7 is a QA mirror, not the official projection engine. It can reproduce key LEAP-side calculations in Python to help give an idea of what results may look like before running LEAP and to validate that the LEAP model is correctly implementing the intended logic. Note that there will always be some differences between the mirror and LEAP results due to differences in calculation order, rounding, and any LEAP-specific features that are not mirrored in Python. The goal is not to get perfect matches but to ensure that the mirror and LEAP results are reasonably close and that any large discrepancies are flagged for review.

### 4.7.2	Responsibilities

Module 7 can:

- run a Python mirror of the stock-turnover calculation;

- calculate mirrored vehicle-km and energy;

- accept tidy LEAP output when provided;

- populate comparison columns without overwriting LEAP results;

- flag large differences for review.

### 4.7.3	Outputs

Module 7 produces T13_mirror_outputs and T13_mirror_fuel_outputs, with Python mirror stock, vehicle-km, and energy values for QA. LEAP comparison values are added only when separate LEAP outputs are available to the dashboard or validation workflow.

## 4.8	What researchers can adjust in LEAP

Some variables are suitable for direct LEAP scenario editing because they are useful levers once the stock-flow inputs have been prepared. These include:

- mileage adjustment factors;

- fuel economy or efficiency adjustment factors;

- technology sales shares, such as ICE, BEV, PHEV, and FCEV shares;

- fuel or Device Share settings where relevant.

These variables are useful for scenario design because they let researchers test changes to travel demand, efficiency, technology uptake, and fuel use.

Scrappage and accelerated-retirement settings are partly represented in the Python workflow, but they are not currently written as routine T11 LEAP import variables. They should therefore be treated as testing or mirror-model inputs until the LEAP hand-off is deliberately extended.

Users should remember that even LEAP-editable variables interact with the stock-flow model. For example, changing sales shares affects future stock gradually through turnover. Changing mileage affects energy demand directly but does not change stock. Changing scrappage affects retirements and may affect replacement sales when it is enabled in a scenario workflow.

The general rule is:

- use the pre-LEAP workflow for difficult preparation, base-year calibration, and structural stock-flow assumptions; and

- use LEAP for official projection runs and controlled scenario changes.

The user will need to be careful not to change other variables in LEAP that are not intended to be scenario levers. For example, changing stock shares or survival assumptions directly in LEAP can have unintended consequences for the stock-flow pathway. The pre-LEAP workflow is designed to prepare those assumptions in a consistent way, so they should generally be left unchanged in LEAP unless the user understands the implications.

## 4.9	Python simulated outputs, LEAP, and official results

LEAP is the official projection platform. Once the road inputs are prepared and imported, LEAP calculates the road transport projection as part of the wider APERC energy system model.

The Python workflow supports LEAP by preparing inputs, reconciling the base year, generating import workbooks, and running optional QA checks. It can also run a mirror calculation to help users understand expected behaviour before or after LEAP is run.

The Python mirror and interface outputs should be called simulated outputs or QA outputs. They are useful because they show the likely effect of assumptions before the full LEAP model is run, and they help users understand whether inputs are plausible. However, the final result still depends on LEAP’s own stock-turnover calculations, scenario expressions, calculation order, and integration with the wider energy system.

This matters because road transport affects the rest of the energy system. Road fuel demand can affect electricity generation, refining, biofuels, hydrogen, oil product demand, imports, exports, and the final energy balance.

A useful way to interpret the workflow is:

Interface = review and update assumptions
Python workflow = prepare, reconcile, and simulate/check road inputs
LEAP = official projection and integration with the whole energy system
Dashboards = compare, diagnose, and explain results

## 4.10	T-Table Lineage and Output Inventory

The workflow names intermediate tables with T* labels. These labels are stable diagnostic and handoff names; they do not always match the module number.

The workflow uses T* table names as stable labels for intermediate outputs and diagnostics. These names are mainly useful when reading dashboards, checking outputs, or discussing where a result came from. They should not be treated as modelling concepts that users need to memorise. In plain English, the T tables show the path from source assumptions to LEAP-ready inputs: base-year branch data, stock targets, sales and turnover, sales shares, fuel allocation, reconciliation scalars, Device Shares, LEAP import rows, diagnostics, and optional Python mirror outputs.

When a dashboard result looks unusual, the T tables help identify where the issue entered the workflow. For example, a problem may come from the original stock/mileage/fuel economy assumptions, the passenger or freight stock projection, the survival and turnover calculation, future sales-share assumptions, the allocation of ESTO fuel totals, reconciliation scalars, or differences between the Python mirror and LEAP results.

The table below is the practical lineage from model inputs to LEAP import.

| Table | Produced by | Primary source inputs | What it contains | Main saved file |
| --- | --- | --- | --- | --- |
| T4_base_year_branches | Module 2 | Module 1 canonical long CSV, vehicle taxonomy, branch/fuel mappings | One base-year row per model branch/fuel combination. It carries economy, scenario, transport type, vehicle type, drive type, size, fuel, LEAP branch path, base stock, mileage, fuel economy, source/provenance flags, and fill/split diagnostics. | results/<economy>/module2/T4_base_year_branches.csv |
| T5_stock_targets | Module 3 | T4 base stocks, population, GDP, ESTO road energy proxy, Module 1 stock shares and growth controls | Vehicle-type stock targets by year. Passenger rows include motorisation, saturation, GDP-per-capita income elasticity, saturation flag, and vehicle-equivalent weight diagnostics. Freight rows include GDP index, final elasticity, raw elasticity, clamping flag, energy/GDP growth rates, elasticity adjustment, source, and note. | results/<economy>/module3/T5_stock_targets.csv |
| T5_stock_targets_pre_reconciliation | Workflow copy before Module 6 re-anchoring | Original Module 3 T5 | Snapshot of stock targets before Module 6 changes the base-year stock anchor. This is the conceptual Module 3 output and is used by the normal Module 3 dashboard view when present. | results/<economy>/module3/T5_stock_targets_pre_reconciliation.csv |
| T5_stock_targets_post_reconciliation | Module 6 stock re-anchoring plus Module 3 table shape | T5 pre-reconciliation, T9 reconciled base-year stock | Stock target paths re-anchored to reconciled base-year stock. Passenger keeps the original final-year physical stock target and fades the base adjustment to zero. Freight keeps the original growth index from the reconciled base stock, so GDP-elasticity growth shape is unchanged while physical stock levels move. | results/<economy>/module3/T5_stock_targets_post_reconciliation.csv |
| T6_sales_turnover | Module 4 | T5 stock targets, Module 1 survival curves, vintage profiles, turnover/scrappage controls | Vehicle stock-flow table by year: target stock, new sales, natural retirements, additional retirements, final stock, stock above target flags, scale factors, and explicit scrappage values when supplied. | results/<economy>/module4/T6_sales_turnover.csv |
| T6_sales_turnover_pre_reconciliation | Workflow copy before Module 6 re-anchoring | Original Module 4 T6 | Snapshot of sales and turnover based on the pre-reconciliation T5. This is the conceptual Module 4 output used by the normal stock/sales dashboard when present. | results/<economy>/module4/T6_sales_turnover_pre_reconciliation.csv |
| T6_sales_turnover_post_reconciliation | Module 4 rerun after Module 6 re-anchoring | T5 post-reconciliation and the same survival/vintage/turnover settings | Sales and turnover recalculated from the re-anchored stock target path. This is the active table used by downstream Module 6/7 outputs after re-anchoring. | results/<economy>/module4/T6_sales_turnover_post_reconciliation.csv |
| T6v_vintage_profiles | Module 4 | Module 1 vintage profile rows and survival settings | Base-year vintage/age distribution used by the turnover model. It is stored separately because it is an age-profile table rather than a year-by-year stock-flow table. | results/<economy>/module4/T6v_vintage_profiles.csv |
| T6v_vintage_profiles_pre_reconciliation | Workflow copy before Module 6 re-anchoring | Original Module 4 T6v | Snapshot paired with T6_sales_turnover_pre_reconciliation. Usually identical in structure and often values, but kept so the pre-reconciliation dashboard can use a complete Module 4 pair. | results/<economy>/module4/T6v_vintage_profiles_pre_reconciliation.csv |
| T6v_vintage_profiles_post_reconciliation | Module 4 rerun after Module 6 re-anchoring | Same inputs as T6v plus rerun context | Vintage profile output paired with T6_sales_turnover_post_reconciliation. | results/<economy>/module4/T6v_vintage_profiles_post_reconciliation.csv |
| Lifecycle profile workbooks | Lifecycle profile exporter | Final active T6v | LEAP lifecycle profile files with Area:, Profile:, blank separator, and Year/Value rows. The export includes survival and vintage profile workbooks, a manifest, and a ZIP for interface download. | results/<economy>/lifecycle_profiles/ |
| T7_sales_shares | Module 5 | T4 base-year branches, EV sales data, Module 1 base-year sales share rows, fallback rules | Base-year sales shares by economy, scenario, transport type, vehicle type, and drive type. These shares anchor future drive-type mix. | In memory and diagnostics; saved only where configured or surfaced in dashboard outputs. |
| T7f_future_shares | Module 5 | Explicit future sales-share file, Module 1 projected sales-share rows, 9th-edition trajectory scaling, or flat fallback | Future sales-share trajectories by year, vehicle type, and drive type. Method/source flags distinguish explicit input, Module 1 projected rows, shape-preserving scaling, hold-flat, and fallback behavior. | In memory and diagnostics; saved only where configured or surfaced in dashboard outputs. |
| T8_fuel_allocation | Module 6 | T4 base-year branch energy, ESTO road fuel totals, fuel eligibility config, PHEV liquid/electric split | Provisional allocation of observed ESTO fuel totals to eligible model branches before scalar reconciliation. It records allocation method, priority/spillover behavior, branch eligibility, and allocated energy. | results/<economy>/module6/T8_fuel_allocation.csv |
| T9_reconciliation_scalars | Module 6 | T8 allocated fuel energy, T4 initial stock/mileage/efficiency, Module 1 reconciliation weights and scalar bounds | Branch/fuel-level reconciliation table. It contains initial energy, ECF, stock/mileage/efficiency scalars, adjusted stock, adjusted mileage, adjusted efficiency, final branch fuel energy, bound flags, iteration flags, and branch metadata. | results/<economy>/module6/T9_reconciliation_scalars.csv |
| T10_device_shares | Module 6 | T9 reconciled branch/fuel values | LEAP Device Share rows calculated after reconciliation. Device Share is fuel energy allocation expressed as a share within each technology branch after adjusted stock/mileage/efficiency are known. | results/<economy>/module6/T10_device_shares.csv |
| T11_leap_ready | Module 6 | T9 reconciled base-year values, T10 Device Shares, T6 sales/stock-flow outputs, T7/T7f sales shares | Canonical LEAP-ready long table. It includes branch path, variable, scenario, year, value, scale, units, dimensions, and source metadata for Stock, Sales, Mileage, Fuel Economy, Device Share, Sales Share, and Stock Share. Current Accounts rows are appended after Module 6 by taking the base-year Target slice and relabeling the scenario. | results/<economy>/module6/T11_leap_ready.csv |
| T12_reconciliation_diagnostics | Module 6 | T9 final fuel energy and ESTO fuel totals | Fuel-level QA table: pre/post model energy, ESTO target, gap, gap percent, ECF summary, scalar-bound status, reconciliation status, and validation flags. | results/<economy>/module6/T12_reconciliation_diagnostics.csv |
| T12_phev_utilisation_diagnostics | Module 6 | PHEV branches from T9, configured PHEV utilisation, electricity/liquid energy split | PHEV electric/liquid split QA. It back-calculates electric driving share from reconciled energy and compares it with the configured utilisation rate. | results/<economy>/module6/T12_phev_utilisation_diagnostics.csv |
| T13_mirror_outputs | Module 7 | T6 active sales/turnover, T9 base technology assumptions, T10 Device Shares, T7f sales shares | Optional Python mirror of LEAP-like stock, vehicle-km, and energy calculations for validation and comparison against LEAP outputs when available. | results/<economy>/module7/T13_mirror_outputs.csv |
| T13_mirror_fuel_outputs | Module 7 | T13 mirror outputs and T10 Device Shares | Optional fuel-level mirror outputs and fuel energy comparisons. | results/<economy>/module7/T13_mirror_fuel_outputs.csv |

### 4.10.1	Runtime naming rule for pre/post reconciliation

The conceptual outputs of Modules 3 and 4 are pre-reconciliation T5, T6, and T6v. During a full workflow, Module 6 can re-anchor stock trajectories to reconciled base-year stock and rerun Module 4. When this happens, the workflow keeps the original tables under explicit *_pre_reconciliation names and replaces the active downstream tables with post-reconciliation versions:

Module 3/4 original:
  T5, T6, T6v

Before re-anchoring, saved as:
  T5_pre_reconciliation
  T6_pre_reconciliation
  T6v_pre_reconciliation

After re-anchoring:
  T5 = T5_post_reconciliation
  T6 = T6_post_reconciliation
  T6v = T6v_post_reconciliation

If no Module 6 stock re-anchoring occurs, plain T5, T6, and T6v are still the original pre-reconciliation tables. The normal dashboard page module3.html is intended to show the original pre-reconciliation stock, sales, and turnover view. The separate module3_post_reconciliation.html page is intended to show only charts whose values changed after stock re-anchoring.

## 4.11	Validation Requirements

Before data are passed to LEAP, the workflow should check:

- no duplicate records in key output tables;

- required dimensions are populated;

- stock, mileage, fuel economy, sales, and shares are non-negative where required;

- sales shares sum to 1 within each relevant group;

- Device Shares sum correctly within each technology branch;

- base-year fuel totals match ESTO within tolerance after reconciliation;

- PHEV utilisation diagnostics remain within configured tolerance;

- scalar bounds and fallback assumptions are reported;

- large stock, mileage, or efficiency adjustments are flagged for review.

## 4.12	Main Files

Runtime files:

- codebase/road_workflow.py

- codebase/adapters/road_module1_defaults.py

- codebase/modules/module2_base_year.py

- codebase/modules/module3_stock_targets.py

- codebase/modules/module4_sales_turnover.py

- codebase/modules/module5_sales_shares.py

- codebase/modules/module6_reconciliation_and_leap_handoff.py

- codebase/modules/module7_mirror.py

Support files:

- scripts/generate_module1_defaults.py

- codebase/config/workflow_defaults.yaml (runtime workflow switches and paths)

- codebase/config/model_defaults.yaml (guidance-only calibration reference; not a runtime fallback source)

- codebase/config/fuel_mappings.yaml

- codebase/schemas/

- codebase/diagnostics/

## 4.13	Relationship to Other Documents

Use this guide for implementation sequence and module responsibilities for Modules 2-7. It is the source of truth for Module 2-7 behavior.

Use road_transport_model_methodology.md for the shorter conceptual guide.

Use multinode_road_module1_repo_guide.md in road_model_inputs_interface/docs/new model/ for Module 1 behavior. That guide is the source of truth for Module 1 data sourcing, the CSV contract, the static row contract, researcher UI workflow, and versioning. The Module 1 sections in this document summarize the handoff contract only; always check that guide for full Module 1 details.

Use transition_audit_report.md only for historical migration context. It is not the current implementation source of truth.

## 4.14	Appendix A: Road Branch Tree and Module 1 Measures

This appendix summarizes the implemented road branch structure and the Module 1 measures required at each level of the tree. It is intended as a compact reference for diagrams, static-contract checks, and handoff discussions.

The taxonomy comes from codebase/config/vehicle_mappings.yaml and codebase/config/fuel_mappings.yaml. The measure placement reflects the current Module 1 static contract used by the interface. One current mismatch is worth noting: the model taxonomy allows FCEV for Motorcycles and LCVs, but the current static contract does not require Motorcycles\FCEV or LCVs\FCEV rows.

Demand

  Passenger road
    Measures:
      Passenger Vehicle Saturation
      Passenger Saturation Reached
      Passenger Stock Growth Rate Adjustment
      Reconciliation Weight Stock
      Reconciliation Weight Mileage
      Reconciliation Weight Efficiency
      Reconciliation Bound Lower Mileage
      Reconciliation Bound Upper Mileage
      Reconciliation Bound Lower Efficiency
      Reconciliation Bound Upper Efficiency
      PHEV Electric Driving Share

    Turnover calibration
      Measures:
        Turnover Rate Bound Lower
        Turnover Rate Bound Upper

    Age <0-37>
      Measures:
        Survival Rate
        Vintage Profile Share

    LPVs [sizes: small, medium, large]
      Measures:
        Stock
        Stock Share
        Sales Share
        Vehicle Equivalent Weight

      ICE <size>
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG,
          Biogasoline, Biodiesel, Biogas, Efuel
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      HEV <size>
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Motor gasoline, Gas and diesel oil,
          Biogasoline, Biodiesel, Efuel
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      EREV <size>
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Electricity, Motor gasoline, Biogasoline, Efuel
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      PHEV <size>
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Electricity, Motor gasoline, Biogasoline, Efuel
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      BEV <size>
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Electricity
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      FCEV <size>
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Hydrogen
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

    Motorcycles
      Measures:
        Stock
        Stock Share
        Sales Share
        Vehicle Equivalent Weight
        Vehicle Equivalent Weight Lower Bound
        Vehicle Equivalent Weight Upper Bound

      ICE
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG,
          Biogasoline, Biodiesel, Biogas, Efuel
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      BEV
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Electricity
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      FCEV
        Note:
          Valid in model taxonomy, but not currently present in the static contract.

    Buses
      Measures:
        Stock
        Stock Share
        Sales Share
        Vehicle Equivalent Weight
        Vehicle Equivalent Weight Lower Bound
        Vehicle Equivalent Weight Upper Bound

      ICE
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG,
          Biogasoline, Biodiesel, Biogas, Efuel
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      BEV
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Electricity
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      FCEV
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Hydrogen
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

  Freight road
    Measures:
      Freight GDP Elasticity Adjustment
      Reconciliation Weight Stock
      Reconciliation Weight Mileage
      Reconciliation Weight Efficiency
      Reconciliation Bound Lower Mileage
      Reconciliation Bound Upper Mileage
      Reconciliation Bound Lower Efficiency
      Reconciliation Bound Upper Efficiency
      PHEV Electric Driving Share

    Turnover calibration
      Measures:
        Turnover Rate Bound Lower
        Turnover Rate Bound Upper

    Age <0-37>
      Measures:
        Survival Rate
        Vintage Profile Share

    Trucks [sizes: medium, heavy]
      Measures:
        Stock
        Stock Share
        Sales Share

      ICE <size>
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG,
          Biogasoline, Biodiesel, Biogas, Efuel
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      BEV <size>
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Electricity
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      FCEV <size>
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Hydrogen
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

    LCVs
      Measures:
        Stock
        Stock Share
        Sales Share

      ICE
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Motor gasoline, Gas and diesel oil, Natural gas, LPG, LNG,
          Biogasoline, Biodiesel, Biogas, Efuel
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      PHEV
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Electricity, Motor gasoline, Biogasoline, Efuel
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      BEV
        Measures:
          Stock Share
          Sales Share
        Fuels:
          Electricity
        Fuel-level measures:
          Mileage
          Fuel Economy
          Mileage correction factor
          Fuel Economy correction factor

      FCEV
        Note:
          Valid in model taxonomy, but not currently present in the static contract.

## 4.15	Appendix B: Module 1 Measures by Branch Level

This appendix describes the same Module 1 measure placement without dictating the actual branch taxonomy. Use it when designing diagrams or validation logic that should apply to branch levels rather than to specific vehicle, drive, or fuel names.

Transport level
  Example path shape:
    Demand\<Passenger road or Freight road>
  Measures:
    Passenger-only:
      Passenger Vehicle Saturation
      Passenger Saturation Reached
      Passenger Stock Growth Rate Adjustment
    Freight-only:
      Freight GDP Elasticity Adjustment
    Passenger and freight:
      Reconciliation Weight Stock
      Reconciliation Weight Mileage
      Reconciliation Weight Efficiency
      Reconciliation Bound Lower Mileage
      Reconciliation Bound Upper Mileage
      Reconciliation Bound Lower Efficiency
      Reconciliation Bound Upper Efficiency
      PHEV Electric Driving Share

Turnover-calibration level
  Example path shape:
    Demand\<transport>\Turnover calibration
  Measures:
    Turnover Rate Bound Lower
    Turnover Rate Bound Upper

Age-profile level
  Example path shape:
    Demand\<transport>\Age <0-37>
  Measures:
    Survival Rate
    Vintage Profile Share

Vehicle-type level
  Example path shape:
    Demand\<transport>\<vehicle type>
  Measures:
    Stock
    Stock Share
    Sales Share
    Vehicle Equivalent Weight
    Vehicle Equivalent Weight Lower Bound
    Vehicle Equivalent Weight Upper Bound
  Notes:
    Vehicle Equivalent Weight bounds are currently used for some passenger
    vehicle types. Freight vehicle types currently require Stock, Stock Share,
    and Sales Share only.

Drive or drive-size level
  Example path shapes:
    Demand\<transport>\<vehicle type>\<drive>
    Demand\<transport>\<vehicle type>\<drive> <size>
  Measures:
    Stock Share
    Sales Share

Fuel level
  Example path shapes:
    Demand\<transport>\<vehicle type>\<drive>\<fuel>
    Demand\<transport>\<vehicle type>\<drive> <size>\<fuel>
  Measures:
    Mileage
    Fuel Economy
    Mileage correction factor
    Fuel Economy correction factor

Condensed rule of thumb:

Transport level:
  growth controls, reconciliation controls, turnover controls, age profiles

Vehicle-type level:
  Stock, Stock Share, Sales Share, and selected vehicle-equivalent weights

Drive or drive-size level:
  Stock Share and Sales Share

Fuel level:
  Mileage and Fuel Economy